***********************************

ZAS Distribution Disk Documentation

***********************************

CP/M Version 1.1

Copyright 1981 Western Wares

P. O. Box C

Norwood, CO  81423

This disk contains the CP/M command files which

comprise the ZAS Development Package:

ZAS.COM - Z8000 Cross Assembler

ZLK.COM - Object Task Builder

ZLD.COM, ZLDX.COM - Absolute Object Loader

ZEX.COM - Z8000 Run Time Execution Support

Source files useful for CP/M and ZEX interfacing

from Z8002 code are also on the disk:

ZEXCP.LIB - This file defines fixed symbols

relating to CP/M functions and

locations within ZEX (e.g. PSA, USERSC).

It has been written to be compatible

with both ZAS (using $INCLUDE) and

MAC (or RMAC) using MACLIB.

Use this file when creating hardware

dependent patches to ZEX based on the

"ZNULCP" source files.

ZEXZ.LIB - This file is convenient to "$include"

in Z-8000 application routines which

will be using CP/M facilities accessed

through the ZEX run time modules. It

contains symbol definitions for CP/M

functions, and system call identifiers

for ZEX.

CPMINT.ZAS - Z8002 routines to perform basic

I/O functions through ZEX to CP/M.

In addition, the null hardware dependent patches

for ZEX and ZLD are included in source form:

ZNULCP.ASM - 8080 Assembly language routine

containing null versions of the

SETSEG, SETUPZ and RUNZ routines.

(See User Manual for details)

ZNULCP.ZAS - Z8000 Assembly language routine

containing a null version of the

HOSTZ routine. (See User Manual)

ZLDNULCP.ASM - 8080 Assembly language routines

which can be modified to support

arbitrary program loading functions.

These are patched into ZLD and ZLDX.COM.

To install ZEX, the user should modify the first two

of these files for the particular CPU bus master switching circuitry

used, as described in the Alternate Processing section of the User Manual.

CP/M's utility DDT is convenient for performing the

patches into ZEX (and the patches for ZLD and ZLDX).

For example, assuming you have created and assembled

new versions of ZNULCP.ASM and ZNULCP.ZAS,

named ZNEW.ASM and ZNEW.ZAS, producing files

ZNEW.HEX and ZNEW.ZRL, the base address (%1xx) for the HOSTZ routine

contained in ZNEW.ZRL can be determined by computing the

next even address following the 8080 routines in ZNEW.HEX.

The last location for HOSTZ (%1yy) should not exceed %13F.

Then, assuming ZNEW.ZAB has been created from ZNEW.ZRL,

A>ZLK ZNEW.ZAB|=ZNEW(*)/%1xx  ,

the following steps can be used to create your

version of ZEX.COM:

1. A>ZLD ZNEW.ZAB 5000  ; Load the output of ZLK offset by %5000

2. A>DDT ZEX.COM

-F103,13F,0		; Clear out patch area

-IZNEW.HEX		; Read your version (8080)

-R

-M51xx,51yy,1xx	; Move Z-8000 code to the patch area

-S10A

10A 00 01		; Set up pointer to HOSTZ (01xx)

10B 00 xx (even!)	; low byte of pointer

10C 00 .

-^C

3. A>SAVE 10 ZEXNEW.COM	; Save your custom version of ZEX

Notice that once your version of ZEX is created,

an entire memory image, consisting of ZEX (%100-%FFF),

and a Z8000 application program (%1000 to base of CCP),

can be SAVEd for later execution from the CP/M CCP.

For example, assuming you have assembled the

sample memory test program, MTST.ZAS, and ZLK'd it with

CPMINT:

A>ZLK MTST.ZAB;=MTST,CPMINT(*)/1006 ,

The following steps will produce the file ZMTST.COM,

which can be executed on your Z-8002 from CP/M:

1. A>ZLD MTST.ZAB	(load the absolute file)

2. A>ZEX ^A		(load your version of ZEX.COM,

the ^A keeps CP/M from

?			  executing ZEX)

3. A>SAVE 20 ZMTST.COM	(save the memory image,

20 pages is sufficient

for MTST, and is determined

from the ZLK MAP output)

************************************

As an example of Z8000 program development using ZAS,

the Z-8000 portion of ZEX is included (ZMNCPM.ZAS).

This code is located at address %500, and can be modified

by the user to change the normal operating conditions for ZEX,

as described below.

To assemble and locate ZMNCPM, type:

A>ZAS ZMNCPM/X/S

A>ZLK /S/MS @ZMNCPM

The resulting absolute code may now be loaded into

memory at location %5500,

A>ZLD ZMNCPM.ZAB 5000

Now, DDT can be used to display the code:

A>DDT ZEX.COM

DDT VERS x.x

NEXT  PC

0800 0100

-D5500

5500 76 0F 0F FC .... etc.

-D500

0500 76 0F 0F FC .... (should be the same as above)

-M5500,5AFF,500  (Move it to the correct spot in ZEX)

ZEX can be configured to run with either the Z-8001 or the Z-8002.

The version of ZEX.COM contained on this disk has been configured for

the Z-8002. To create a Z-8001 version of ZEX, change the symbol

'Z8001' to TRUE in ZMNCPM.ZAS, then 'ZAS', 'ZLK' and 'ZLD' it as

described above. The ZMNCPM.ZLK file should be modified to indicate

the segment for ZEX and ZMNCPM.

Finally, move it to the proper spot in ZEX.COM with DDT, as shown above.

The command,

A>SAVE 10 YOURZEX.COM

will complete the configuration.

The default FCW for programs executed with ZEX is determined

from the INITIAL_FCW symbol within ZMNCPM.ZAS, and is currently set

to start user code running in non-segmented, system mode.

Application programs should be located above %1006 for

execution under ZEX, using System Call instructions to

access the functions in ZMNCPM.

See the User Manual for more details on ZEX.

***************************************

Configuring ZLD.COM and ZLDX.COM:

These two programs have been written to allow simple

user modification to support widely varying program loading requirements.

The area from %103 to %1FF within ZLD and ZLDX is reserved for

user modification, and consists of a jump vector to several user routines.

The file ZLDNULCP.ASM explains the function of each routine, and is the

default version currently contained within ZLD.COM and ZLDX.COM.

By modifying this file, ZLD can be set up to load to segmented memory

with separate Program and Data areas, for example. Downloading to a

prototype circuit is also possible.

***************************************

Please report any bugs (or questions) to:

Western Wares

P. O. Box C

Norwood, CO   81423

(303) 327-4898

If a bug is suspected, send a sample which exhibits

the problem, together with the .COM files to Western Wares

on a single density 8" disk.

We also require that the original distribution disk be returned

for any updates or bug fixes.

Welcome to 16-bit Microcomputing!

*********************************

###############