If you are using PC Elevator, you must set-up your disk with Vfeature so that

all volumes are smaller than 33 megabytes.  You must also modify the "device="

statement in your CONFIG.SYS file to read "device=fixt_drv.sys /E".

If you are using the older ORCHID Turbo-Board or a similar product, all disk

volume sizes must be smaller than 33 megabytes.  You must also append to your

TURBO.SYS file the line "device=fixt_drv.sys /T".  (This file may have some

other name for non-ORCHID products.)