--- C4 LIBRARY FUNCTION MANUAL ---

settime(month, day, year, hour, minute, second)

char month, day, year, hour, minute, second;

Sets display clock to parameters passed. Time is in 24 hour format.

Ranges:

month : 1-12		day : 1-31		year : 00-99

hour : 0-23		minute : 0-59		second : 0-59

char gettime(select)

char select;

returns binary value of clock/calendar reading of selected field.

'select' has the range of 0-7. 6 & 7 however, return 0.

Values of 'select' are:

0: month (1-12)		1: day (1-31)		2: year (0-99)

3: hour (0-23)		4: minute (0-59)	5: second (0-59)

6: 0			7: 0

int setbaud(baud_rate)

int baud_rate;

Sets RS-232 port to 'baud_rate' specified.

Baud rates can be one of the following:

75, 110, 150, 300, 600, 1200, 2400, 4800, 9600

If a baud rate of 0 is specified, then the last programmed baud

rate is returned. For example:

pnum(baud(0))

will output the current baud rate to the display.

seta(range, channel)

char range, channel;

Sets range to be used on subsequent reada() functions.

Valid ranges are:

0 : x1, 1 : x5, 2 : x50, 3 : x250

Ranges are processed modulo 4, so if 5 is specified, a gain of

x5 will be selected for the channel.

Channel takes the range of 0 through 15, modulo 16, and corresponds

to the 16 analog input channels.

The following is a table of range specifications:

GAIN	RESOLUTION	FULL SCALE

x1	2.5 mv		+- 4.9975 v

x5	500 uv		+- 0.9995 v

x50	50  uv		+- 99.95 mv

*  x250	10  uv		+- 19.99 mv

* This range is probable range to use for temperature measurements.

int reada(channel)

char channel;

Returns 2s complement value of the analog voltage at 'channel' input.

'channel' must be in the range of 0-15.

See also 'seta()' above for range setting to use with this function.

int temp(channel)

char channel;

Returns 2s complement value of the analog voltage present at 'channel'

input, converted to degrees celsius +- 10%.

'channel' must be in the range of 0-15.

Temperature resolution is +- 1 degree c and has the range of +- 460

degrees c.

A call to setc() function is required before using this function,

however, setc() need only be called once for a particular thermo-

couple type. This is true because the values are stored in non-volatile

memory and can be guaranteed to be the same upon subsequent power ups.

Therfore, setc() could be called from the editor/loader mode.

int setc(gain, constant1, constant2, channel)

char gain, constant1, constant2, channel;

Sets up the gain to be used upon subsequent temp() calls to the

'channel', and the conversion constants to be applied to the voltage

read from 'channel'.

Channel 0 is the reference ambient channel & must be read with a

'reada() function. i.e.:

ambient = reada(0) / 4;  /* this is approximate and may need to be

changed if the OMEGA zero pot is changed  */

Also, set up channel 0 before using temp(). :

setc(1,1,1,0);	/*  set up channel 0 to gain = x5 */

The following is a suggested table of constants to use. All use a

gain of x250 (setc(3,c1,c2,channel)).:

tc type		constant 1	constant 2

-------		----------	----------

J		52		52

K		20		40

T		20		42

E		30		63

R		3		6

S		3		6

outa(value, channel)

char value, channel;

Outputs 'value' in 12 bit offset form to analog output 'channel'.

'channel has the range of 0-15.

examples:

0 = -5v

800h = 0v

fffh = +5v

don(channel)

char channel;

Outputs a logic '1' to digital channel 'channel'.

'channel' relates to the 3 parallel ports and the bit number resolves

to the channel number modulo 8.

Example:

don(9)

Turns on bit 1 (of 0-7) of port 1 (0-2).

In other words, 'channel' / 8 yields the port number and 'channel'

mod 8 yields the bit number.

doff(channel)

char channel;

Outputs a logic '0' to the digital channel 'channel'.

Ports and bits are computed as specified in 'don()' above.

write(byte)

char byte;

Writes 'byte' to rs-232 serial port.

See 'setbaud()'.

close()

Outputs an end-of-file character (ctrl-z, 1ah) to the serial port.

Use after the last character is written.

#######################################################################