SPERRY ITs with two hard disks attached may have problems addressing the second

drive if it has a different number of heads (data surfaces) than the first.

This problem may manifest itself as an excessive number of errors reported dur-

ing high level formatting of the second drive.  If you experience this problem,

modify the CONFIG.SYS file on your working diskette to append a "/u" to the

"device=fixt_drv.sys" statement (device=fixt_drv.sys /u).  Then reboot with the

working diskette, rerun VOL_DEF and reformat the second drive.  You should also

be sure that the hard disk CONFIG.SYS file has the "/u" modification.