Concurrent DOS Release Note                     HDMAINT Command

HDMAINT - THE CP/M HARD DISK MAINTENANCE COMMAND

To use your hard-disk system with any operating system,  you

must first reserve and prepare space on your hard disk. This

space  is   called  a "partition".  HDMAINT is a  hard  disk

maintenance program for CP/M disk partitions.

HDMAINT  is  a hard-disk maintenance utility that creates  a

partition on your hard disk and prepares it for CP/M program

and text files.  This CP/M type of format allows the use  of

passwords on files.

The  companion utility FDISK prepares a partition using  the

DOS  type  of disk format.  Concurrent  will  recognise  and

support both types of disk format (DOS or CP/M).

Syntax:

HDMAINT

Explanation:

HDMAINT prepares your  hard  disks,   quickly  tests   them,

and  examines  or changes an existing hard-disk  partition's

options.

HDMAINT  is  largely self-explanatory.    After   you   read

"Using  HDMAINT,"  you  can  enter the starting command  for

HDMAINT  and follow the menus HDMAINT  displays  to  perform

the   hard-disk  maintenance  task  of   your  choice.   The

menus you see when you run HDMAINT on your own system  might

not be  the  same  as  the menus  reproduced  here;  HDMAINT

displays only the menus your system can use.

HDMAINT is only used for CP/M partitions. It shows partitions

created   by  any other operating systems (including DOS)  as

reserved for "unknown use."   (The only exception to this  is

that HDMAINT can make a bootable DOS partition non-bootable).

HDMAINT can prepare and maintain one CP/M partition on  each

hard  disk  in your system.   HDMAINT does not allow you  to

create two CP/M partitions on  a  single  hard disk.

Concurrent  checks the status of a hard disk's  partitioning

1

only  when   it  first  loads.    If  you  change  a  disk's

partitioning with HDMAINT,  you must reload Concurrent after

you  exit  from  the HDMAINT program.

Using HDMAINT

HDMAINT  presents  commands  in  a  series   of   menus.  To

select  a command,  press the function key that  corresponds

to  the command  you  want.  After  you  select  a  command,

HDMAINT   prompts  you  for any information it  requires  to

perform the selected task.

Select  any  command  (except exit) at the  Main   Menu   to

display  the  partition  summary.    This  summary  includes

information  about the hard disk including the total  number

of cylinders  and  the number  already  assigned,   location

and  system  type of each partition, and which partition, if

any, is bootable.

HDMAINT Main Menu

When  you enter the  HDMAINT  command,   the  HDMAINT   Main

Menu appears on your screen. Below is a list of the Main Menu

commands and the task they perform.

Command         Tasks

F1 - Display or change disk partitioning

Select  this command to create,  change,   or

delete a CP/M partition.

F3 - Verify a hard disk partition

Select this command to scan a CP/M partition,

testing  and  verifying that all data can  be

read without errors.

F5 - Display or change bootstrap behaviour

Select  this command to switch  the  bootable

partition   on  the  first hard disk,  or  to

write Concurrent system loader information to

a CP/M partition on the first hard disk.

F7 - Display or change a partition's options

Select  this command to display or  change  a

CP/M    partition's   directory   label,   to

increase    the    number    of     directory

entries,  or  to  change the status  of   the

partition  read-after-write verify switch.

ESC - Exit from this program

2

Select this command to leave HDMAINT.

How to Display or Change Disk Partitioning: Summary

HDMAINT  allows you to set up a CP/M partition for the first

time,  or to change the existing one,  on each hard disk  in

your  system.  Remember that if you are changing an existing

CP/M  partition,  you  #must#  copy  all  the  files  in  that

partition  onto  floppy  disk.  The process  of  changing  a

partition  always destroys all the files on  the  partition,

and so you need to take a back-up.

Setting  up  or  changing   a  disk  partition  involves  the

following  steps,  which are described in more detail in  the

next section:

1    Take  back up copies of any files on an existing  CP/M

partition.

2    Enter the HDMAINT program

3    Delete any existing partition

4    Define the new CP/M partition by choosing:

the hard disk cylinder number at which the partition starts

the partition size

the number of directory entries to be allowed

whether to include an automatic verification cycle

5    Tell  HDMAINT actually to create the partition you  have

defined.

6    HDMAINT then:

creates the new partition

tests for faults on the disk ("bad blocks")

writes  information on a reserved area on the hard  disk

to  tell  the system that the partition exists (this  is

called "enabling" the partition).

3

7    Restart  your  system  and the new  partition  comes  into

operation.

How to Display or Change Disk Partitioning: Step-by-Step Guide

Below  are  step-by-step instructions for changing  the  CP/M

partition  on your hard disk,  or creating one for the  first

time.

1 Take back up copies of any files on an existing CP/M partition.

(You  only  need  this stage if you  have  a  CP/M  partition

already  on your hard disk.) Take copies of all the files  on

the CP/M partition that you are going to change.  Use COPY or

PIP  for  this (see the manual if you're not sure how to  use

COPY or PIP).

2 Enter the HDMAINT program

Put the program disk containing HDMAINT  in drive A and type:

A>HDMAINT

This  leads  to the Main Menu.  Choose F1 -Display or  change

disk partitioning and you will see the following options:

F3 - Change CP/M partition      F5 - Delete CP/M partition

ESC - Return to main menu

3 Delete any existing partition

(You  only  need  this  stage  if you  already  have  a  CP/M

partition  on  your hard disk.) Remember  that  deleting  the

partition  erases all files on the partition.  Make sure  you

have a backup of all the files on the CP/M partition and then

press F5 to delete the partition.

HDMAINT asks:

Do you really want to proceed?

Press  F9  for  "yes" and HDMAINT erases  the  partition  and

returns  you to the Main Menu.  Press Esc for "no".  From the

Main  Menu you need to select F1 again  and then  proceed  to

stage 4, below.

Note that HDMAINT can only delete the CP/M partitions on your

hard disks;  it can't delete DOS partitions or any other kind

or partition.

4

4 Define the new CP/M partition

To  create  a new CP/M partition or to change   an   existing

CP/M  partition,    select    F3.  This leads to a  new  menu

screen.

In  this  new  menu  screen,  HDMAINT   proposes  a  starting

cylinder  number  that  is always the first cylinder  of  the

largest  available  space on the disk  (Concurrent  considers

hard disks to be divided into "cylinders",  each of which has

a   unique  number).     If  this   starting   cylinder    is

acceptable,   press F10.

To  specify a different starting cylinder number,  follow the

menu instructions to type the starting cylinder  number   you

want.   You  can enter up to three digits.  At least 10  free

cylinders  must follow  the  starting  cylinder you   choose.

When   you  are satisfied  with  the  new  starting  cylinder

number, press <ENTER>.   Press F10 to continue.

You  specify  the  partition  size in much  the  same   way.

HDMAINT calculates  the  available  disk space and  proposes

a  partition  size  in  cylinders.   It  also  displays  the

approximate   size  of the  partition,   in  kilobytes.   If

the  partition size HDMAINT proposes  is  acceptable,  press

F10.

To specify a different partition size,  type a  new  number,

0-999.   When  you  are  satisfied  with the partition size,

press <ENTER> to update the partition size message  and  the

partition summary display.  Press F10 to proceed.

You  can specify partitions as large as the  maximum  number

of  cylinders   your   hard  disk  allows.   HDMAINT   never

proposes  a partition  size  larger  than   8192  kilobytes.

This is because CP/M-86 cannot access CP/M partitions larger

than this.   If you enter  a  partition  size  too large for

CP/M-86 to access, the highlighted message

CCP/M only

appears on the screen.

HDMAINT  displays  the  CP/M Options Menu next.   This  menu

allows  you  to  change  the  number  of  directory  entries

assigned   to   the  CP/M partition and the  status  of  the

read-after-write  verify switch.

A  CP/M  partition  can  have  two   possible   numbers   of

directory  entries.   If   your  CP/M  partition   has   the

smaller  number  of directory entries,  you can  double  the

number   with   the   HDMAINT  "Display   or   Change   CP/M

5

options" command.  You cannot reduce the number of directory

entries  once  you set the directory size  to   the   larger

value  unless  you  back  up your  files,  delete  the  CP/M

partition,  and create a new partition of  the  same   size.

Initial   partition  log-in  time  increases  slightly  when

you specify the maximum number of directory entries.

A  number  of  considerations  affect  whether  you   choose

the smaller  or  larger  number  of  directory entries.  The

larger number requires more disk space,  reducing the  space

for   file storage.   It  is,   however,  a good choice  for

users who plan to store a large number of short  files.   It

is also a good  choice for  those  who  use  date  and  time

stamping  as  date and time stamps use approximately 1/4  of

the total number  of  directory entries.  The smaller number

of directory entries is preferable for users needing maximum

space  for  file  storage and those with  fewer  but  longer

files.

To  verify all write operations to the  CP/M  partition   of

the   hard    disk,    you   can   enable   read-after-write

verification.   This option results in greater data security

but requires more  time for Concurrent to write to the  hard

disk.

HDMAINT highlights the current value of each option.   Press

the appropriate  function  key  to  change  to the alternate

value.

5 Tell HDMAINT actually to create the partition you have defined

When  you  are satisfied with your choices,  press   Esc   to

enter the values and continue.

Note  that  a  change in either the directory  entry  number

option  or  read-after-write  verification  option does  not

take  effect until the next system power-up or reset.  These

two options can also be changed by selecting F7, "Display or

change a partition's options" from the Main Menu.

6 HDMAINT then creates the partition

The  process  of creating a partition takes  time  (about  2

seconds per  cylinder).   To  abort  this  operation,  press

Esc.  The  message

Do you really want to stop?

appears  on  the  screen.   Press  Esc again  to  abort  the

operation and   return  to  the  Main  Menu.   To   continue

creating  the partition, press F9.

6

HDMAINT   performs  several  operations  when   it    creates

a  partition.   Below is a list of these operations  and  the

screen messages that accompany them.

Message         Meaning

Creating new partition

HDMAINT   checks   critical  areas  of    the

partition before proceeding to ensure that  a

partition  can  be  created at  the  location

specified.   No  defects are allowed in  this

area.

Testing cylinder nnn

nnn bad blocks found (will be ignored by system)

HDMAINT  displays the number (nnn)  of   each

cylinder   as   it tests whether data can  be

written  to and read from each area  of   the

partition   being  created.   As defects  are

found,  HDMAINT  notes the defective   blocks

and  displays  the total number (nnn) of  bad

blocks.    HDMAINT    automatically    avoids

these   defective   areas   during  system NT

operation.

Enabling partition

HDMAINT   writes  information    about    the

partition   in   a reserved area on the  hard

disk.   The  partition does not exist   until

this step is completed.

The  beginning  of   every  partition  must   be   free   of

defects   because   HDMAINT   reserves   that  area  for  an

operating system's use.   If HDMAINT finds  any  bad  blocks

in   that  area  while creating  a   partition,   it   stops

and    informs   you   that  the  reserved  system  area  is

defective.   If   this  occurs,   you  can press   Esc    to

end   the  creation  process.   Under  normal circumstances,

press F7 to respecify the partition.

If you press Esc,  HDMAINT does not create a new   partition

and returns you to the Main Menu.

To respecify the partition,  press F7.   This option returns

you  to   the  Enter  Starting  Cylinder  Menu.   You   must

repeat  the entire process of selecting a starting  cylinder

and selecting a partition size.  If possible, HDMAINT offers

new  default  values to avoid  the  defective  area.   These

values   may   be   completely different   from   those  you

originally  specified because HDMAINT always proposes values

7

that    result   in   the   largest   possible   defect-free

partition.   HDMAINT  does not,  however,  prevent you  from

specifying  other values even if they include areas  HDMAINT

found  defective  earlier in the session.   HDMAINT displays

the message

Creation of partition completed

when  it  has  avoided defective  areas   and   successfully

created  the  partition.   Press Esc to return to  the  Main

Menu.

7 Restart your system and the new partition comes into operation

The  change  or  create  operation  has  now  been  completed

successfully. You need to restart your system so that you can

begin to use the new partition.

How to Verify a Hard-Disk Partition

The  HDMAINT Main Menu also allows you to verify a hard  disk

partition.  If  you  are having  problems  particularly  when

saving   files   and want  to  determine  whether  your  hard

disk is damaged or your application program is at fault,  you

should try verifying  your hard-disk partitions.

Select  "F3  - Verify a hard disk partition" from the   Main

Menu to  scan  a partition.  This process identifies any new

hard-disk surface defects that occur after the partition  is

created without destroying your files.

You  can  stop  the  verification process at  any  time   by

pressing Esc.  HDMAINT pauses and displays the message

Do you really want to stop?

To  continue  the  verification,  press  F9.   To  stop  the

procedure,  press Esc,  and HDMAINT returns you to the  Main

Menu.

When HDMAINT has tested all cylinders, the message

Verification complete

appears.   Press  Esc  to return to the HDMAINT  Main  Menu.

Press F9 to verify the partition again.

If  HDMAINT  locates  a large number   of   additional   bad

blocks,  repeat   the  verification.   If  the number of bad

blocks varies widely on repeated verifications,  a  hardware

failure,   rather than  damage to the hard-disk surface,  is

the most likely cause of the problem.  In this case, consult

you computer dealer.

8

If you cannot access a CP/M partition on the hard  disk, use

this command to verify that the system area is not  damaged.

If  HDMAINT  finds  a  defect in  the   system   area,   the

following message appears:

reserved system area is defective

If  the verification procedure reveals a defective  area  on

your  hard   disk,   back  up  all your hard-disk  files  on

floppy disks with the COPY or PIP command.  (You can use the

verify  option  with  the PIP  command  to  determine  which

files,  if  any,  have been affected by the defective area.)

You can also check  the  files individually.  Select F1 from

the  Main  Menu  to repartition your hard  disk  around  the

defective area,  then restore  your  backed up files to  the

hard disk.

How to Display or Change Bootstrap Behaviour

Select "F5 - Display or change  bootstrap  behaviour"   from

the Main  Menu  to change your personal computer's operating

system loading behaviour. The following are the options that

appear  when you select this command:

F2 - Change bootable partition

F4 - Enable booting CCP/M from CP/M partition

F6 - Enable booting CCP/M from DOS partition

ESC - Return to main menu

Press  F2 to select which partition on your first hard  disk

you  want to be bootable. The following options appear:

F1 - Change bootable partition

ESC - Return to Main Menu

Press  F1   to  show  the  currently   bootable   partition.

Keep  pressing  F1  until  the  partition  of your choice is

shown as bootable.  Press Esc to return to the Main Menu.

Press  F5  from  the  Main Menu.   The  Display  or   Change

Bootstrap  Behaviour    Menu   reappears.   Select   F4   to

write  Concurrent  system  loader information  to  the  CP/M

partition  on the  first  hard  disk.

Finally,  copy  the file CCPM.SYS to user number 0  on   the

9

CP/M  partition  or the root directory of the DOS  partition

(depending  on your previous choice).   Use PIP or COPY  for

this purpose.

How to Display or Change a Partition's Options

The  command "F7 - Display or change a partition's  options"

from  the  Main Menu controls several additional  functions.

When  you press F7,  the Display or  Change  a   Partition's

Options  Menu appears:

F2 - Display or Change Directory labels

F4 - Display or Change CP/M options

ESC - Return to main menu

Select  "F2 - Display or Change Directory labels," to display

the current directory  labels  or  to assign a new  directory

label.   When  you  select  F2,   the   Display   or   Change

Directory Labels Menu appears:

F1 - Change CP/M Directory label

ESC - Return to main menu

To  create  a  directory label or to  change  a  partition's

existing   directory     label,    press   the   key    that

corresponds  to  the partition you want to change.

When  you select a partition,  a label entry  menu  appears.

This menu  gives  you  guidelines for acceptable label names

for  the  selected  partition.   To create a  new  directory

label,  type   the  name you want to  enter.   As  you  type

characters,  they  appear  on your screen  within  quotation

marks.

When you are satisfied with the label name you typed,  press

<ENTER>.  HDMAINT  enters the new label name and returns you

to  the Display or Change Directory labels   Menu.    Notice

that your new directory label now appears at the top of your

screen. If you only pressed  <ENTER>  you  created  a  blank

directory label.  A blank directory label is acceptable.

You  can repeat the label entry operation as many times   as

you  like.   When   the  new  directory label  displayed  is

acceptable,  press Esc to return  to  the  Main  Menu.    To

create  another directory label, press the appropriate key.

HDMAINT  cannot change a CP/M partition's directory label if

the label is password-protected.

10

HDMAINT Program Exit

Select  "ESC - Exit from this program" from the  Main   Menu

when  you   want  leave  HDMAINT.   If  HDMAINT  created  or

deleted any partitions during  operation,   a  program  exit

menu   appears.   HDMAINT   requires   you  to  reload   the

operating system from disk at this time.   This step ensures

that    the   operating   system  notices   any    hard-disk

partitioning   changes  you  made  with HDMAINT  because  it

checks    information    about   hard-disk  partitions  only

when it loads.

HDMAINT Errors

If  HDMAINT  detects   a   hard-disk   error   that    makes

continued  operation  impossible,   a message appears in the

status  line  at the bottom of your  screen,  and   a   beep

sounds.   Press  D  to display details of the error.

HDMAINT  then displays the details of the hard-disk error at

the top  of  the program exit menu.   You must insert a boot

disk in drive A and press F9 to reload the operating system.

HDMAINT  displays  hard-disk  error messages  only  when  it

detects  a   hardware  failure.   Such  messages   are   not

ordinarily the result of operator or program errors.

Examples

The  following  examples assume a computer  with  one   305-

cylinder  hard    disk.    Remember  that   HDMAINT   starts

only  when  the following conditions exist:

*    Your hard disk is powered up and ready to operate.

*    The correct version of Concurrent is loaded.

Example 1

This   example  then  creates a  60-cylinder  bootable  CP/M

partition starting at cylinder 0,  marks it  as  "bootable,"

and writes the system loader information to it.  This leaves

cylinders 60 through 305 free for other use.

1    Insert  the HDMAINT program disk in drive  A.   At  the

prompt, enter the command:

A>HDMAINT

11

1    From the Main Menu,  select "F1 -  Display  or   change

disk partitioning."

2    To specify the partition type,  at the next menu select

"F3 - Change CP/M partition."

3    In  response  to  the  question  "Do  you  really  want

to proceed?", press F9 to delete the existing portion.

4    At  the Enter Starting Cylinder Menu,  press   F10   to

accept the default value HDMAINT proposes.

5    At  the  Partition  Size  Menu,  type  60   and   press

<ENTER>.   When   the  screen  is  updated to show  the

value you entered, press F10 to continue.

6    At  the  CP/M Options Menu,  press Esc to  accept   the

default values HDMAINT proposes.

7    When HDMAINT finishes creating the partition, press Esc

to return to the Main Menu.

8    Select "F5 - Display or change bootstrap behaviour" from

the Main Menu.

9    At   the  next  menu,   select  "F2  - Change  bootable

partition."

10   When the next menu appears,  press F1 as required until

the  message   (bootable) is displayed beside the  CP/M

partition description in the partition summary.   Press

Esc to return to the Main Menu.

11   At  the  Main  Menu,  press "F5  - Display  or   change

bootstrap behaviour."   Select  F4  to write  Concurrent

system loader information to the CP/M partition.  Press

Esc to  go  back to the Main Menu.

12   At  the  Main  Menu,  select "Esc  - Exit   from   this

program."  When  the  exit  menu  appears,  insert  the

Concurrent  boot  disk in drive A.   Press F9 to  leave

HDMAINT and reload the operating system.

13   Copy  the file CCPM.SYS from a Concurrent boot disk  to

the new partition.

If  no  disk is in drive A at the next  system  power-up  or

reset, Concurrent automatically loads from the hard disk.

Example 2

This example creates a 245-cylinder partition  starting   at

cylinder  60.   Cylinders  0 through 59 are already reserved

12

for another partition as shown in Example 1.

1    Insert  the  disk  containing the  HDMAINT  program  in

drive  A. At the prompt, type

A>HDMAINT

2    From the Main Menu,  select "F1  -  Display  or  change

disk partitioning."

3    To specify the partition type,  at the next menu select

"F3 - Change CP/M partition."

4    At  both the Starting  Cylinder  and   Partition   Size

menus,  press  F10 to accept the default values HDMAINT

proposes.

5    When  HDMAINT  finishes creating the  partition,  press

Esc  to return to the Main Menu.

6    At  the  Main Menu,  select "Esc  -  Exit   from   this

program."  When   the   exit  menu  appears,  insert  a

system boot disk in drive A, and press F9.

Example 3

This  example updates the system loader information   in   a

CP/M partition  created with other versions of Concurrent or

CP/M-86 to enable loading this version of Concurrent.

1    Insert  the  disk  containing the  HDMAINT  program  in

drive  A. At the prompt, type

A>HDMAINT

and press <ENTER>.

2    From  the  Main Menu,  select "F5 - Display  or  change

bootstrap behaviour."

3    At the next menu,  select "F4 -  Enable  booting  CCP/M

from CP/M partition."

4    When  HDMAINT  finishes writing the  Concurrent  system

loader, press Esc to return to the Main Menu.

5    Press Esc to leave HDMAINT.

6    Update   the  operating  system  file  by  copying  the

file CCPM.SYS  from the new Concurrent boot disk to the

existing CP/M partition.

13

At  the  next  system  power-up  or  reset,   this   version

of  Concurrent   automatically  loads  from  the  hard  disk

when the following conditions exist:

No disk is in drive A.

The CP/M partition contains no CPM.SYS file.

14

############