This  file  has  a  list of files  on  the  distribution  floppies  and

directions  for installing them manually.   These directions should  be

used if the 'install' program will not work on your computer, or if you

have  special  requirements  not satisfied by  the  'install'  program.

Please  try the 'install' program first.   If you are not familiar with

the concepts of DOS tree structured directories, environment variables,

or  execution  paths,  please refer to the DOS manual  for  instruction

prior to attempting to install your software manually.

Please note that not all of these floppies  may have been supplied with

your product, and that the name of the  EMULATOR PC INTERFACE  floppies

may differ (eg: 68000 PC INTERFACE); however,  the directory structures

and  file contents are  still applicable.   In  particular,  the  CROSS

ASSEMBLER/LINKER and USER DEFINABLE ASSEMBLER floppies are not supplied

with all products.  Discs  3  and  4  of EMULATOR PC INTERFACE  are not

supplied  with  emulators  which  do  not support  the  optional 16 bit

external logic analyzer.

CONTENTS OF DISTRIBUTION DISCS:

EMULATOR PC INTERFACE Disc 1:

\     (root directory)

-------------

/ install.exe

/   setup.txt, product.txt

/     readme.doc  (this file)

/

HP64700     (sub-directory)

------------

/

/

BIN        (sub-directory)

-------------

PCxxx.1         (First  part of emulator  user  interface,

too  large  for a single floppy.  Will be

combined on hard disc.)

PCEXPR.X        (Host  commands  file  downloaded  to  the

emulator to extend the base command set.)

PCxxxxxx.xxx    (Optional  command files downloaded to the

emulator to extend the base command set.)

RHP64000.EXE    (File format reader for HP 64000 format.)

Rxxxxxxx.EXE    (File format reader for optional formats.)

EMULATOR PC INTERFACE Disc 2:

\     (root directory)

-------------

/  install.bat

/

HP64700         (sub-directory)

------------

/          \

/              \

BIN            TABLES   (sub-directories)

-------------    -------------

PCxxx.2          64700TAB    (Default emulator device table.)

(Part 2)

EMULATOR PC INTERFACE Disc 3:

\     (root directory)

-------------

/  install.bat

/

HP64700     (sub-directory)

------------

/

/

BIN        (sub-directory)

-------------

PTxxx.1    (First  part of emulator  user  interface  which

supports  timing  analysis,  too  large  for a

single floppy.   Will be combined on hard disc.)

EMULATOR PC INTERFACE Disc 4:

\     (root directory)

-------------

/  install.bat

/

HP64700         (sub-directory)

------------

/          \

/              \

BIN            TABLES   (sub-directories)

-------------    -------------

PTxxx.2          64700TAB    (Default emulator device table.)

(Part 2)

CROSS ASSEMBLER/LINKER (HP648xx) Disc 1:

(Note that file: '\system\648xx\productr' contains revision/build info)

\     (root directory)

-------------

/  install.bat

/

HP64700     (sub-directory)

------------

\

\

TABLES      (sub-directory)

-------------

Axxxxx   (Assembler tables for xxxxx processor)

Lxxxxx   (Linker tables for xxxxx processor)

USER DEFINABLE ASSEMBLER (HP64851) Disc 1:

(Note that file: '\system\64851\productr' contains revision/build info)

\     (root directory)

-------------

/  install.bat

/

HP64700     (sub-directory)

------------

/

/

BIN                (sub-directory)

-------------

asm.exe         Assembler table interpreter

lnk.exe         Linker table interpreter

read64.exe      Formatted dump for HP64000 files

Optional HP64700 series software disc:

(Note that file: '\system\64xxx\productr' contains revision/build info)

\     (root directory)

-------------

/  install.bat

/

HP64700     (sub-directory)

------------

/

/

BIN                (sub-directory)

-------------

XXXXXXXX.exe    Optional hp64700 software product

YYYYYYYY.exe    Optional hp64700 software product

The  general idea of installation is to get the files needed  onto  the

hard  disc.   The files must be placed into sub-directories on the hard

disc as follows:

<tld>       (Top level directory, your choice of

------------    name. Install defaults to: \HP64700 )

/          \

/              \

BIN            TABLES      (sub-directories)

-------------    -------------

asm.exe          Axxxxx         (Note that multiple processors

lnk.exe          Lxxxxx          may be supported.  The linker

PCxxx.exe        64700TAB        and assembler tables, and the

PCyyy.exe        Ayyyyy          emulator  user  interface are

PTxxx.exe        Lyyyyy          needed for each.)

XXXXX.exe

In  addition  to  creating the directory structure on the  target  hard

disc, and copying the files required, the environment variable HPTABLES

must be assigned the path name of the TABLES directory,  this path will

always be the TABLES subdirectory of your top level directory  (<tld>).

This  variable is used by the emulator,  assembler,  and linker to find

the required tables.   The DOS 'set' command is used to initialize this

variable  (ex:  set HPTABLES=c:\HP64700\TABLES) and you  will  probably

want  to place this command into your 'autoexec.bat' file for execution

on boot up.  In addition, the HPBIN environment variable must be set to

the directory  containing the host  commands  file  (PCEXPR.X)  and any

optional command files (ex:  set HPBIN=c:\HP64700\BIN).

You should also add the 'BIN' sub-directory to your execution 'PATH' to

allow  execution  of the development programs while you are  developing

software in your own directories.  See the DOS command 'path' for usage

of this command.  (ex: PATH=\;\com;\HP64700\BIN)  You will want to edit

or add this command to your 'autoexec.bat' file also.

Follow these steps to install your software manually:

o   Decide upon a directory name for your top level directory. <tld>

o   Build the directory structure if it does not already exist.

md <tld>

md <tld>\BIN

md <tld>\TABLES

o   Copy  the  'PCxxx.1'  file from the  EMULATOR PC INTERFACE  disc 1

'\HP64700\BIN' directory to the file: <tld>\BIN\PCxxx.1

(example: copy a:\HP64700\BIN\pcz80.1 c:\HP64700\BIN\*.1)

o   Copy  the  'PCxxx.2'  file from the  EMULATOR PC INTERFACE  disc 2

'\HP64700\BIN' directory to the file: <tld>\BIN\PCxxx.2

(example: copy a:\HP64700\BIN\pcz80.2 c:\HP64700\BIN\*.2)

o   If  your  emulator has an external 16 bit logic analyzer,  and you

wish  to  perform  timing  analysis  with  it,  you  will  need to

install PTxxx.EXE.  If  installing  PTxxx.EXE,  copy the 'PTxxx.1'

file  from  the  EMULATOR PC INTERFACE  disc  3  '\HP64700\BIN'

directory to the file: <tld>\BIN\PTxxx.1

(example: copy a:\HP64700\BIN\ptz80.1 c:\HP64700\BIN\*.1)

o   If  installing  PTxxx.EXE,   copy  the  'PTxxx.2'  file  from  the

EMULATOR PC INTERFACE  disc  4  '\HP64700\BIN'  directory  to  the

file: <tld>\BIN\PTxxx.2

(example: copy a:\HP64700\BIN\ptz80.2 c:\HP64700\BIN\*.2)

o   If  you  do  not  already  have  a  emulator  device  table   file

(64700TAB) that you have set up and want to keep, copy the default

device  table  file:  \HP64700\TABLES\64700TAB  from the  EMULATOR

PC INTERFACE disc 2 to the file: <tld>\TABLES\64700TAB

(example: copy a:\HP64700\TABLES\64700TAB c:\HP64700\TABLES)

o   Combine the two parts of the emulator executable as follows:

( /B options on copy command specify required binary copies )

cd \<tld>\BIN                      <== On destination drive.

copy PCxxx.1/B+PCxxx.2/B PCxxx.exe/B

del PCxxx.1

del PCxxx.2

o   If  installing  PTxxx.EXE,  combine the two  parts of the emulator

executable with timing as follows:

( /B options on copy command specify required binary copies )

cd \<tld>\BIN                      <== On destination drive.

copy PTxxx.1/B+PTxxx.2/B PTxxx.exe/B

del PTxxx.1

del PTxxx.2

o   Copy  all files from the '\HP64700\TABLES' directory on the  CROSS

ASSEMBLER/LINKER disc  1 to the:  '<tld>\TABLES' directory on  the

hard disc.  (ex: copy a:\HP64700\TABLES\*.* c:\HP64700\TABLES)

o   Copy  '\SYSTEM\648xx\PRODUCTR'  from  the  CROSS  ASSEMBLER/LINKER

disc  1 to the file: '<tld>\648xx.REV'.   This file is a text file

containing revision information  for  the  CROSS ASSEMBLER/LINKER.

The 'xx' refers to the product number for the processor and may be

determined by  examining  the product  number on the floppy  disc.

o   Copy  all  files  from the '\HP64700\BIN' directory on  the   USER

DEFINABLE ASSEMBLER disc  1 to the:  '<tld>\BIN' directory  on the

hard disc.  (ex: copy a:\HP64700\BIN\*.* c:\HP64700\BIN)

o   Copy  '\SYSTEM\64851\PRODUCTR'  from  the USER DEFINABLE ASSEMBLER

disc  1 to the file: '<tld>\64851.REV'.   This file is a text file

containing revision information  for the USER DEFINABLE ASSEMBLER.

o   Copy all files from the '\HP64700\BIN' directory on any additional

HP64700 family discs supplied to the: '<tld>\BIN' directory on the

hard disc.

(ex: copy a:\HP64700\BIN\*.* c:\HP64700\BIN)

o   Edit  your  autoexec.bat  file to add or edit the 'set HPTABLES=',

'set HPBIN=' and 'PATH=' commands as discussed above.

o   Reset the computer to set up environment and check operation.