MTOS-UX Demonstration Usage:  Release 1.0           120888

The included software package is an example of how a multi-tasking

environment is implemented under MTOS-UX.

In this example there are 5 tasks:

1) CO:      This task is responsible for the initiation of

the i/o ports, creation and start-up of the

other tasks, creation of all message buffers

and clean up at shut down. ( CO stands for coordinator )

2) PORT:    This task polls all the i/o ports initiated in this

system: AUX.2, Basic Rate port A, Basic Rate port B

and the keyboard.  When it detects something at one

of these ports it notifies the specified task.

3) DSP:     This task displays the strings other tasks request

to have displayed.  In this example each string displayed

on the screen is also sent out on AUX.2 by DSP.

4) MA1:     This task is the analysis task for basic rate port A.

MA1 receives the buffer received on port A from 'PORT'

it then does what it wants with this buffer and

sends the display request ( or send on AUX.2 ) of

what it choses to 'DSP'.  In this example MA1 only

notifies 'DSP' of a message receipt. This can however

easily be expanded to suite another need.

5) MA2:     This task does the same as the above only it

interacts with port B.

On the disc included in this package ther are 14 files:

co.c        the body of the 'CO' task (  main() )

port.c      the body of the 'PORT' task

dsp.c       the body of the 'DSP'  task

ma.c        the bodies of 'MA1' and 'MA2'

util.c      send and receive functions used by all tasks

linkcom.c   initiation functions for basic rate setup

links.c     configuration of link connections needed

tcd.c       initiation structures for each task2

mainsym.h   global and useful symbols

tos.h       events and corresponding structures

task initiation structures

paval.h     symbols used to initiate AUX drivers

err.o       error handling functions

makefile

sw.doc      the documentation you are now reading

It should be noted that each task started by 'CO' receives

a pointer to a structure where it finds its configuration

information.

Each task also starts by initiating itself ( initiation function

to be found at the end of each task file) and exchanging

START_REQ and START_CONF messages with 'CO'.

To star-up in your environment read the configuration section.

CONFIGURATION.

To run this package in your environment you need to

edit two files:

1)  co.c

2)  links.c

The co.c files handles the initiation of the basic rate

ports by calling a function named setup().

setup(PORTA,...,...,...,...,nt_power);

.

.

.

setup(PORTB,...,...,...,...,nt_power);

The parameter nt_power should be set to the value required

by the TE:s used.  The CHAMELEAN Basic Rate Library explains

the meaning of the different values.

The links.c file specifies which links are to be handled

in this perticular case. The structure linksA defines the

links on port A and linksB the links on port B.  The structure

must be terminated with the link NONE,NONE,NONE.

The structure has the following appearance: ( defined in mainsym.h)

typedef struct

{

int type;    /* specifies whether the TEI value has to be set */

int tei;     /* TEI value                                     */

int sapi;    /* SAPI value                                    */

} LINK;