# 5421 Emulation Support

Due to the silicon problem in the 5421, we are not able to provide full debugging capabilities when debugging code in shared memory.  We have attempted to include software workarounds for the known silicon problems with limited success.  I have listed the features that have no known software workarounds and remain “broken” in the latest 54x_shared.dvr file.  I have also listed ways to debug code running on the 5421 given the debugger’s limited capabilities.

## Major features that work

- Single stepping in each core.

- Reading and writing to shared memory.

- Shared memory breakpoint support.

- Synchronous running.

- Synchronous stepping of the same code (i.e. stepping the same instructions in shared memory on both cores)

## Features that don’t work

- Synchronous stepping – If two instructions with different cycle execution times are stepped synchronously, the potential exists for a core to finish it’s instruction and halt (asserting XSTOP) while the second core is still executing it’s multi-cycle instruction.  If the executing instruction tries to access shared memory in the other core’s subsystem, the instruction will not execute properly.

- Synchronous Halting – If a synchronous halt is issued, the cores will finish executing the current instruction before stopping. If two instructions with different cycle execution times are being executed, the potential exists for a core to finish it’s instruction and halt (asserting XSTOP) while the second core is still executing it’s multi-cycle instruction.  If the executing instruction tries to access shared memory in the other core’s subsystem, the instruction will not execute properly.

- Global Breakpoints – Global breakpoints are implemented when a core hits a break event and drives the EMU0/1 pin low causing the other core to halt.  There is a several cycle delay between the time the first core halts and the second core halts due to the EMU0/1 pin being driven low.  The potential exists for a core to cause a break event, halt (asserting XSTOP) and drive the EMU0/1 pin low.  The second core will continue executing for several cycles before halting in response to the EMU0/1 signal.  If the core executes an instruction that accesses shared memory in the other core’s subsystem, the instruction will not execute properly.