A.  Serial Flash loader Rev 2.x for TMX320F206 DSP from Texas instruments.

Refer to Readme.pdf shipped with utilities rev 2.0 for detailed documentation

B.1 Flash loader main screen

An example for menu selections in F206sldr.exe

a. 	Enter the directory path for the Level 2 algorithm code COFFA.out file to be loaded

b. 	Enter the directory path for the customer flash code COFF file to be programmed

c. 	Define the SFLAG bits to enable flash array and flash functions. SFLAG is a 16 bit number, each of these bits will select the required flash functions in the serial loader Level 2.

( Select SFLAG bits to be 0xe007 for fast programming of Flash 0 and 1 array on  new F206 devices. This selection will Clear, Erase flash 0 array and program Flash0 and Flash 1 arrays. Flash 1 array comes in erased state, ready for programming. Flash 0 is cleared and erased to remove the Level 1 loader code shipped with the device).Refer to section E for SFLAG command details in Readme.pdf.

d. 	Select the host time out limit to 25 seconds if 32k words of flash are to be programmed

e. 	Define flash code buffer size in SARAM to be 2048.

f. 	Select 'Send A character to initiate load'. This will make the host send a character 'a' before loading

g. 	Select 'Level 2 COFF file only' if the serial loader is to load only Level 2 code to do CLEAR and ERASE. Enabling will make the utility ignore customer COFF file selection.

h.   Select 'Continuous operation' if the serial loader is to run in the background

continuously. This is a useful feature for programming multiple F206 boards without

any menu selection on the host terminal.

After selecting the above options, the selections can be saved in a configuration file for  future use. The File menu provides all the standard file operations.

B.2 Flash loader communication status window

The host serial loader software will update a communication status window while the loading in progress. The status window will display the selected COM port, percentage of loading done, and error message with (error code).

a.    Loading Done            		(1)

b. 	Communication error 	     	(2)

c. 	Algorithm error  in CLEAR		(3)

d.   Algorithm error  in ERASE             (4)

e.   Algorithm error  in PROGRAM      (5)

B.3 Flash serial loader 'Configure ports' menu

Serial port options

Select 'Configure ports' menu to define the serial ports (COM1,COM2,

COM3 or COM4)

B.4  Flash loader serial port selections

Select the baud rate as desired and 8 bits data, no parity and 1 stop bit