XFree86 Video Timings HOWTO

Eric S. Raymond <esr@thyrsus.com>

Version 3.0, 8 Aug 1997

Abstract

How to compose a mode line for your card/monitor combination under

XFree86.  The XFree86 distribution now includes good facilities for

configuring most standard combinations; this document is mainly

useful if you are tuning a custom mode line for a high-performance

monitor or very unusual hardware.  It may also help you in using

xvidtune to tweak a standard mode that is not quite right for your

monitor.

1.  Disclaimer

You use the material herein SOLELY AT YOUR OWN RISK.  It is possible to harm

both your monitor and yourself when driving it outside the manufacturer's

specs. Read Overdriving Your Monitor (section 11., page 1) for detailed cau-

tions. Any damages to you or your monitor caused by overdriving it are your

problem.

The most up-to-date version of this HOWTO can be found at the Linux Documen-

tation Project <URL:http://sunsite.unc.edu/LDP> web page.

Please direct comments, criticism, and suggestions for improvement to

esr@snark.thyrsus.com. Please do not send email pleading for a magic solution

to your special monitor problem, as doing so will only burn up my time and

frustrate you -- everything I know about the subject is already in here.

2.  Introduction

The XFree86 server allows users to configure their video subsystem and thus

encourages best use of existing hardware.  This tutorial is intended to help

you learn how to generate your own timing numbers to make optimum use of your

video card and monitor.

We'll present a method for getting something that works, and then show you

how you can experiment starting from that base to develop settings that opti-

mize for your taste.

Starting with XFree86 3.2, XFree86 provides an XF86Setup(1) program that

makes it easy to generate a working monitor mode interactively, without mess-

ing with video timing number directly.  So you shouldn't actually need to

calculate a base monitor mode in most cases.  Unfortunately, XF86Setup(1) has

some limitations; it only knows about standard video modes up to 1280x1024.

If you have a very high-performance monitor capable of 1600x1200 or more you

will still have to compute your base monitor mode yourself.

Recent versions of XFree86 provide a tool called xvidtune(1) which you will

probably find quite useful for testing and tuning monitor modes.  It begins

with a gruesome warning about the possible consequences of mistakes with it.

If you pay careful attention to this document and learn what is behind the

pretty numbers in xvidtune's boxes, you will become able to use xvidtune

effectively and with confidence.

If you already have a mode that almost works (in particular, if one of prede-

fined VESA modes gives you a stable display but one that's displaced right or

left, or too small, or too large) you can go straight to the section on Fix-

ing Problems with the Image (section 14., page 1).  This will enlighten you

on ways to tweak the timing numbers to achieve particular effects.

If you have xvidtune(1), you'll be able to test new modes on the fly, without

modifying your X configuration files or even rebooting your X server.  Other-

wise, XFree86 allows you to hot-key between different modes defined in Xcon-

fig (see XFree86.man for details).  Use this capability to save yourself has-

sles!  When you want to test a new mode, give it a unique mode label and add

it to the end of your hot-key list.  Leave a known-good mode as the default

to fall back on if the test mode doesn't work.

3.  How Video Displays Work

Knowing how the display works is essential to understanding what numbers to

put in the various fields in the file Xconfig.  Those values are used in the

lowest levels of controlling the display by the XFree86 server.

The display generates a picture from a series of dots.  The dots are arranged

from left to right to form lines.  The lines are arranged from top to bottom

to form the picture.  The dots emit light when they are struck by the elec-

tron beam inside the display.  To make the beam strike each dot for an equal

amount of time, the beam is swept across the display in a constant pattern.

The pattern starts at the top left of the screen, goes across the screen to

the right in a straight line, and stops temporarily on the right side of the

screen.  Then the beam is swept back to the left side of the display, but

down one line.  The new line is swept from left to right just as the first

line was.  This pattern is repeated until the bottom line on the display has

been swept.  Then the beam is moved from the bottom right corner of the dis-

play to the top left corner, and the pattern is started over again.

There is one variation of this scheme known as interlacing: here only every

second line is swept during one half-frame and the others are filled in in

during a second half-frame.

Starting the beam at the top left of the display is called the beginning of a

frame.  The frame ends when the beam reaches the the top left corner again as

it comes from the bottom right corner of the display.  A frame is made up of

all of the lines the beam traced from the top of the display to the bottom.

If the electron beam were on all of the time it was sweeping through the

frame, all of the dots on the display would be illuminated.  There would be

no black border around the edges of the display.  At the edges of the display

the picture would become distorted because the beam is hard to control there.

To reduce the distortion, the dots around the edges of the display are not

illuminated by the beam even though the beam may be pointing at them.  The

viewable area of the display is reduced this way.

Another important thing to understand is what becomes of the beam when no

spot is being painted on the visible area.  The time the beam would have been

illuminating the side borders of the display is used for sweeping the beam

back from the right edge to the left and moving the beam down to the next

line.  The time the beam would have been illuminating the top and bottom bor-

ders of the display is used for moving the beam from the bottom-right corner

of the display to the top-left corner.

The adapter card generates the signals which cause the display to turn on the

electron beam at each dot to generate a picture.  The card also controls when

the display moves the beam from the right side to the left and down a line by

generating a signal called the horizontal sync (for synchronization) pulse.

One horizontal sync pulse occurs at the end of every line.  The adapter also

generates a vertical sync pulse which signals the display to move the beam to

the top-left corner of the display.  A vertical sync pulse is generated near

the end of every frame.

The display requires that there be short time periods both before and after

the horizontal and vertical sync pulses so that the position of the electron

beam can stabilize.  If the beam can't stabilize, the picture will not be

steady.

In a later section, we'll come back to these basics with definitions, formu-

las and examples to help you use them.

4.  Basic Things to Know about your Display and Adapter

There are some fundamental things you need to know before hacking an Xconfig

entry.  These are:

o your monitor's horizontal and vertical sync frequency options

o your video adapter's driving clock frequency, or "dot clock"

o your monitor's bandwidth

The monitor sync frequencies:

The horizontal sync frequency is just the number of times per second the mon-

itor can write a horizontal scan line; it is the single most important

statistic about your monitor.  The vertical sync frequency is the number of

times per second the monitor can traverse its beam vertically.

Sync frequencies are usually listed on the specifications page of your moni-

tor manual.  The vertical sync frequency number is typically calibrated in Hz

(cycles per second), the horizontal one in KHz (kilocycles per second).  The

usual ranges are between 50 and 150Hz vertical, and between 31 and 135KHz

horizontal.

If you have a multisync monitor, these frequencies will be given as ranges.

Some monitors, especially lower-end ones, have multiple fixed frequencies.

These can be configured too, but your options will be severely limited by the

built-in monitor characteristics.  Choose the highest frequency pair for best

resolution.  And be careful --- trying to clock a fixed-frequency monitor at

a higher speed than it's designed for can easily damage it.

Earlier versions of this guide were pretty cavalier about overdriving multi-

sync monitors, pushing them past their nominal highest vertical sync fre-

quency in order to get better performance.  We have since had more reasons

pointed out to us for caution on this score; we'll cover those under Over-

driving Your Monitor (section 11., page 1) below.

The card driving clock frequency:

Your video adapter manual's spec page will usually give you the card's dot

clock (that is, the total number of pixels per second it can write to the

screen).  If you don't have this information, the X server will get it for

you.  Even if your X locks up your monitor, it will emit a line of clock and

other info to standard output.  If you redirect this to a file, it should be

saved even if you have to reboot to get your console back.  (Recent versions

of the X servers all support a --probeonly option that prints out this infor-

mation and exits without actually starting up X or changing the video mode.)

Your X startup message should look something like one of the following exam-

ples:

If you're using XFree86:

Xconfig: /usr/X11R6/lib/X11/Xconfig

(**) stands for supplied, (--) stands for probed/default values

(**) Mouse: type: MouseMan, device: /dev/ttyS1, baudrate: 9600

Warning: The directory "/usr/andrew/X11fonts" does not exist.

Entry deleted from font path.

(**) FontPath set to "/usr/lib/X11/fonts/misc/,/usr/lib/X11/fonts/75dpi/"

(--) S3: card type: 386/486 localbus

(--) S3: chipset:   924

---

Chipset -- this is the exact chip type; an early mask of the 86C911

(--) S3: chipset driver: s3_generic

(--) S3: videoram:  1024k

-----

Size of on-board frame-buffer RAM

(**) S3: clocks:  25.00  28.00  40.00   3.00  50.00  77.00  36.00  45.00

(**) S3: clocks:   0.00   0.00  79.00  31.00  94.00  65.00  75.00  71.00

------------------------------------------------------

Possible driving frequencies in MHz

(--) S3: Maximum allowed dot-clock: 110MHz

------

Bandwidth

(**) S3: Mode "1024x768": mode clock =  79.000, clock used =  79.000

(--) S3: Virtual resolution set to 1024x768

(--) S3: Using a banksize of 64k, line width of 1024

(--) S3: Pixmap cache:

(--) S3: Using 2 128-pixel 4 64-pixel and 8 32-pixel slots

(--) S3: Using 8 pages of 768x255 for font caching

If you're using SGCS or X/Inside X:

WGA: 86C911 (mem: 1024k clocks: 25 28 40 3 50 77 36 45 0 0 79 31 94 65 75 71)

---  ------       -----         --------------------------------------------

|     |            |                 Possible driving frequencies in MHz

|     |            +-- Size of on-board frame-buffer RAM

|     +-- Chip type

+-- Server type

Note: do this with your machine unloaded (if at all possible).  Because X is

an application, its timing loops can collide with disk activity, rendering

the numbers above inaccurate.  Do it several times and watch for the numbers

to stabilize; if they don't, start killing processes until they do.  SVr4

users: the mousemgr process is particularly likely to mess you up.

In order to avoid the clock-probe inaccuracy, you should clip out the clock

timings and put them in your Xconfig as the value of the Clocks property ---

this suppresses the timing loop and gives X an exact list of the clock values

it can try.  Using the data from the example above:

wga

Clocks    25 28 40 3 50 77 36 45 0 0 79 31 94 65 75 71

On systems with a highly variable load, this may help you avoid mysterious X

startup failures.  It's possible for X to come up, get its timings wrong due

to system load, and then not be able to find a matching dot clock in its con-

fig database --- or find the wrong one!

4.1  The monitor's video bandwidth:

If you're running XFree86, your server will probe your card and tell you what

your highest-available dot clock is.

Otherwise, your highest available dot clock is approximately the monitor's

video bandwidth.  There's a lot of give here, though --- some monitors can

run as much as 30% over their nominal bandwidth.  The risks here have to do

with exceeding the monitor's rated vertical-sync frequency; we'll discuss

them in detail below.

Knowing the bandwidth will enable you to make more intelligent choices

between possible configurations.  It may affect your display's visual quality

(especially sharpness for fine details).

Your monitor's video bandwidth should be included on the manual's spec page.

If it's not, look at the monitor's highest rated resolution.  As a rule of

thumb, here's how to translate these into bandwidth estimates (and thus into

rough upper bounds for the dot clock you can use):

640x480             25

800x600             36

1024x768       65

1024x768 interlaced 45

1280x1024      110

1600x1200      185

BTW, there's nothing magic about this table; these numbers are just the low-

est dot clocks per resolution in the standard XFree86 Modes database (except

for the last, which I interpolated).  The bandwidth of your monitor may actu-

ally be higher than the minimum needed for its top resolution, so don't be

afraid to try a dot clock a few MHz higher.

Also note that bandwidth is seldom an issue for dot clocks under 65MHz or so.

With an SVGA card and most hi-res monitors, you can't get anywhere near the

limit of your monitor's video bandwidth.  The following are examples:

Brand                    Video Bandwidth

----------               ---------------

NEC 4D                   75Mhz

Nano 907a           50Mhz

Nano 9080i               60Mhz

Mitsubishi HL6615        110Mhz

Mitsubishi Diamond Scan       100Mhz

IDEK MF-5117             65Mhz

IOCOMM Thinksync-17 CM-7126   136Mhz

HP D1188A           100Mhz

Philips SC-17AS               110Mhz

Swan SW617               85Mhz

Viewsonic 21PS           185Mhz

Even low-end monitors usually aren't terribly bandwidth-constrained for their

rated resolutions.  The NEC Multisync II makes a good example --- it can't

even display 800x600 per its spec.  It can only display 800x560.  For such

low resolutions you don't need high dot clocks or a lot of bandwidth; proba-

bly the best you can do is 32Mhz or 36Mhz, both of them are still not too far

from the monitor's rated video bandwidth of 30Mhz.

At these two driving frequencies, your screen image may not be as sharp as it

should be, but definitely of tolerable quality. Of course it would be nicer

if NEC Multisync II had a video bandwidth higher than, say, 36Mhz.  But this

is not critical for common tasks like text editing, as long as the difference

is not so significant as to cause severe image distortion (your eyes would

tell you right away if this were so).

4.2  What these control:

The sync frequency ranges of your monitor, together with your video adapter's

dot clock, determine the ultimate resolution that you can use.  But it's up

to the driver to tap the potential of your hardware.  A superior hardware

combination without an equally competent device driver is a waste of money.

On the other hand, with a versatile device driver but less capable hardware,

you can push the hardware's envelope a little.  This is the design philosophy

of XFree86.

5.  Interpreting the Basic Specifications

This section explains what the specifications above mean, and some other

things you'll need to know.  First, some definitions.  Next to each in parens

is the variable name we'll use for it when doing calculations

horizontal sync frequency (HSF)

Horizontal scans per second (see above).

vertical sync frequency (VSF)

Vertical scans per second (see above).  Mainly important as the

upper limit on your refresh rate.

dot clock (DCF)

More formally, `driving clock frequency'; The frequency of the

crystal or VCO on your adaptor --- the maximum dots-per-second it

can emit.

video bandwidth (VB)

The highest frequency you can feed into your monitor's video

input and still expect to see anything discernible. If your adap-

tor produces an alternating on/off pattern, its lowest frequency

is half the DCF, so in theory bandwidth starts making sense at

DCF/2. For tolerably crisp display of fine details in the video

image, however, you don't want it much below your highest DCF,

and preferably higher.

frame length (HFL, VFL)

Horizontal frame length (HFL) is the number of dot-clock ticks

needed for your monitor's electron gun to scan one horizontal

line, including the inactive left and right borders.  Vertical

frame length (VFL) is the number of scan lines in the entire

image, including the inactive top and bottom borders.

screen refresh rate (RR)

The number of times per second your screen is repainted (this is

also called "frame rate").  Higher frequencies are better, as

they reduce flicker.  60Hz is good, VESA-standard 72Hz is better.

Compute it as

RR = DCF / (HFL * VFL)

Note that the product in the denominator is not the same as the

monitor's visible resolution, but typically somewhat larger.

We'll get to the details of this below.

The rates for which interlaced modes are usually specified (like

87Hz interlaced) are actually the half-frame rates: an entire

screen seems to have about that flicker frequency for typical

displays, but every single line is refreshed only half as often.

For calculation purposes we reckon an interlaced display at its

full-frame (refresh) rate, i.e. 43.5Hz. The quality of an inter-

laced mode is better than that of a non-interlaced mode with the

same full-frame rate, but definitely worse then the non-inter-

laced one corresponding to the half-frame rate.

5.1  About Bandwidth:

Monitor makers like to advertise high bandwidth because it constrains the

sharpness of intensity and color changes on the screen.  A high bandwidth

means smaller visible details.

Your monitor uses electronic signals to present an image to your eyes.  Such

signals always come in in wave form once they are converted into analog form

from digitized form.  They can be considered as combinations of many simpler

wave forms each one of which has a fixed frequency, many of them are in the

Mhz range, eg, 20Mhz, 40Mhz, or even 70Mhz.  Your monitor video bandwidth is,

effectively, the highest-frequency analog signal it can handle without dis-

tortion.

For our purposes, bandwidth is mainly important as an approximate cutoff

point for the highest dot clock you can use.

5.2  Sync Frequencies and the Refresh Rate:

Each horizontal scan line on the display is just the visible portion of a

frame-length scan.  At any instant there is actually only one dot active on

the screen, but with a fast enough refresh rate your eye's persistence of

vision enables you to "see" the whole image.

Here are some pictures to help:

_______________________

|                       |     The horizontal sync frequency

|->->->->->->->->->->-> |     is the number of times per

|                      )|     second that the monitor's

|<-----<-----<-----<--- |     electron beam can trace

|                       |     a pattern like this

|                       |

|                       |

|                       |

|_______________________|

_______________________

|        ^              |     The vertical sync frequency

|       ^ |             |     is the number of times per

|       | v             |     second that the monitor's

|       ^ |             |     electron beam can trace

|       | |             |     a pattern like this

|       ^ |             |

|       | v             |

|       ^ |             |

|_______|_v_____________|

Remember that the actual raster scan is a very tight zigzag pattern; that is,

the beam moves left-right and at the same time up-down.

Now we can see how the dot clock and frame size relates to refresh rate.  By

definition, one hertz (hz) is one cycle per second.  So, if your horizontal

frame length is HFL and your vertical frame length is VFL, then to cover the

entire screen takes (HFL * VFL) ticks.  Since your card emits DCF ticks per

second by definition, then obviously your monitor's electron gun(s) can sweep

the screen from left to right and back and from bottom to top and back DCF /

(HFL * VFL) times/sec.  This is your screen's refresh rate, because it's how

many times your screen can be updated (thus refreshed) per second!

You need to understand this concept to design a configuration which trades

off resolution against flicker in whatever way suits your needs.

For those of you who handle visuals better than text, here is one:

RR                                      VB

|   min HSF                     max HSF |

|    |             R1        R2  |      |

max VSF -+----|------------/----------/---|------+----- max VSF

|    |:::::::::::/::::::::::/:::::\     |

|    \::::::::::/::::::::::/:::::::\    |

|     |::::::::/::::::::::/:::::::::|   |

|     |:::::::/::::::::::/::::::::::\   |

|     \::::::/::::::::::/::::::::::::\  |

|      \::::/::::::::::/::::::::::::::| |

|       |::/::::::::::/:::::::::::::::| |

|        \/::::::::::/:::::::::::::::::\|

|        /\:::::::::/:::::::::::::::::::|

|       /  \:::::::/::::::::::::::::::::|\

|      /    |:::::/:::::::::::::::::::::| |

|     /     \::::/::::::::::::::::::::::| \

min VSF -+----/-------\--/-----------------------|--\--- min VSF

|   /         \/                        |   \

+--/----------/\------------------------+----\- DCF

R1        R2  \                       |     \

min HSF                |    max HSF

VB

This is a generic monitor mode diagram.  The x axis of the diagram shows the

clock rate (DCF), the y axis represents the refresh rate (RR). The filled

region of the diagram describes the monitor's capabilities: every point

within this region is a possible video mode.

The lines labeled `R1' and `R2' represent a fixed resolutions (such as

640x480); they are meant to illustrate how one resolution can be realized by

many different combinations of dot clock and refresh rate. The R2 line would

represent a higher resolution than R1.

The top and bottom boundaries of the permitted region are simply horizontal

lines representing the limiting values for the vertical sync frequency. The

video bandwidth is an upper limit to the clock rate and hence is represented

by a vertical line bounding the capability region on the right.

Under Plotting Monitor Capabilities (section 15., page 1)) you'll find a pro-

gram that will help you plot a diagram like this (but much nicer, with X

graphics) for your individual monitor.  That section also discusses the

interesting part; the derivation of the boundaries resulting from the limits

on the horizontal sync frequency.

6.  Tradeoffs in Configuring your System

Another way to look at the formula we derived above is

DCF = RR * HFL * VFL

That is, your dot clock is fixed.  You can use those dots per second to buy

either refresh rate, horizontal resolution, or vertical resolution.  If one

of those increases, one or both of the others must decrease.

Note, though, that your refresh rate cannot be greater than the maximum ver-

tical sync frequency of your monitor.  Thus, for any given monitor at a given

dot clock, there is a minimum product of frame lengths below which you can't

force it.

In choosing your settings, remember: if you set RR too low, you will get

mugged by screen flicker.

You probably do not want to pull your refresh rate below 60Hz.  This is the

flicker rate of fluorescent lights; if you're sensitive to those, you need to

hang with 72Hz, the VESA ergonomic standard.

Flicker is very eye-fatiguing, though human eyes are adaptable and peoples'

tolerance for it varies widely.  If you face your monitor at a 90% viewing

angle, are using a dark background and a good contrasting color for fore-

ground, and stick with low to medium intensity, you *may* be comfortable at

as little as 45Hz.

The acid test is this: open a xterm with pure white back-ground and black

foreground using xterm -bg white -fg black and make it so large as to cover

the entire viewable area.  Now turn your monitor's intensity to 3/4 of its

maximum setting, and turn your face away from the monitor.  Try peeking at

your monitor sideways (bringing the more sensitive peripheral-vision cells

into play).  If you don't sense any flicker or if you feel the flickering is

tolerable, then that refresh rate is fine with you.  Otherwise you better

configure a higher refresh rate, because that semi-invisible flicker is going

to fatigue your eyes like crazy and give you headaches, even if the screen

looks OK to normal vision.

For interlaced modes, the amount of flicker depends on more factors such as

the current vertical resolution and the actual screen contents.  So just

experiment.  You won't want to go much below about 85Hz half frame rate,

though.

So let's say you've picked a minimum acceptable refresh rate.  In choosing

your HFL and VFL, you'll have some room for maneuver.

7.  Memory Requirements

Available frame-buffer RAM may limit the resolution you can achieve on color

or gray-scale displays.  It probably isn't a factor on displays that have

only two colors, white and black with no shades of gray in between.

For 256-color displays, a byte of video memory is required for each visible

dot to be shown.  This byte contains the information that determines what mix

of red, green, and blue is generated for its dot.  To get the amount of mem-

ory required, multiply the number of visible dots per line by the number of

visible lines.  For a display with a resolution of 800x600, this would be 800

x 600 = 480,000, which is the number of visible dots on the display.  This is

also, at one byte per dot, the number of bytes of video memory that are nec-

essary on your adapter card.

Thus, your memory requirement will typically be (HR * VR)/1024 Kbytes of

VRAM, rounded up.  If you have more memory than strictly required, you'll

have extra for virtual-screen panning.

However, if you only have 512K on board, then you can't use this resolution.

Even if you have a good monitor, without enough video RAM, you can't take

advantage of your monitor's potential.  On the other hand, if your SVGA has

one meg, but your monitor can display at most 800x600, then high resolution

is beyond your reach anyway (see Using Interlaced Modes (section 12., page 1)

for a possible remedy).

Don't worry if you have more memory than required; XFree86 will make use of

it by allowing you to scroll your viewable area (see the Xconfig file docu-

mentation on the virtual screen size parameter).  Remember also that a card

with 512K bytes of memory really doesn't have 512,000 bytes installed, it has

512 x 1024 = 524,288 bytes.

If you're running SGCS X (now called X/Inside) using an S3 card, and are

willing to live with 16 colors (4 bits per pixel), you can set depth 4 in

Xconfig and effectively double the resolution your card can handle.  S3

cards, for example, normally do 1024x768x256.  You can make them do

1280x1024x16 with depth 4.

8.  Computing Frame Sizes

Warning: this method was developed for multisync monitors.  It will probably

work with fixed-frequency monitors as well, but no guarantees!

Start by dividing DCF by your highest available HSF to get a horizontal frame

length.

For example; suppose you have a Sigma Legend SVGA with a 65MHz dot clock, and

your monitor has a 55KHz horizontal scan frequency.  The quantity (DCF / HSF)

is then 1181 (65MHz = 65000KHz; 65000/55 = 1181).

Now for our first bit of black magic.  You need to round this figure to the

nearest multiple of 8.  This has to do with the VGA hardware controller used

by SVGA and S3 cards; it uses an 8-bit register, left-shifted 3 bits, for

what's really an 11-bit quantity.  Other card types such as ATI 8514/A may

not have this requirement, but we don't know and the correction can't hurt.

So round the usable horizontal frame length figure down to 1176.

This figure (DCF / HSF rounded to a multiple of 8) is the minimum HFL you can

use.  You can get longer HFLs (and thus, possibly, more horizontal dots on

the screen) by setting the sync pulse to produce a lower HSF.  But you'll pay

with a slower and more visible flicker rate.

As a rule of thumb, 80% of the horizontal frame length is available for hori-

zontal resolution, the visible part of the horizontal scan line (this allows,

roughly, for borders and sweepback time -- that is, the time required for the

beam to move from the right screen edge to the left edge of the next raster

line).  In this example, that's 944 ticks.

Now, to get the normal 4:3 screen aspect ratio, set your vertical resolution

to 3/4ths of the horizontal resolution you just calculated.   For this exam-

ple, that's 708 ticks.  To get your actual VFL, multiply that by 1.05 to get

743 ticks.

The 4:3 is not technically magic; nothing prevents you from using a non-

Golden-Section ratio if that will get the best use out of your screen real

estate.  It does make figuring frame height and frame width from the diagonal

size convenient, you just multiply the diagonal by by 0.8 to get width and

0.6 to get height.

So, HFL=1176 and VFL=743.  Dividing 65MHz by the product of the two gives us

a nice, healthy 74.4Hz refresh rate.  Excellent!  Better than VESA standard!

And you got 944x708 to boot, more than the 800 by 600 you were probably

expecting.  Not bad at all!

You can even improve the refresh rate further, to almost 76 Hz, by using the

fact that monitors can often sync horizontally at 2khz or so higher than

rated, and by lowering VFL somewhat (that is, taking less than 75% of 944 in

the example above).  But before you try this "overdriving" maneuver, if you

do, make sure that your monitor electron guns can sync up to 76 Hz vertical.

(the popular NEC 4D, for instance, cannot.  It goes only up to 75 Hz VSF).

(See Overdriving Your Monitor (section 11., page 1) for more general discus-

sion of this issue. )

So far, most of this is simple arithmetic and basic facts about raster dis-

plays.  Hardly any black magic at all!

9.  Black Magic and Sync Pulses

OK, now you've computed HFL/VFL numbers for your chosen dot clock, found the

refresh rate acceptable, and checked that you have enough VRAM.  Now for the

real black magic -- you need to know when and where to place synchronization

pulses.

The sync pulses actually control the horizontal and vertical scan frequencies

of the monitor.  The HSF and VSF you've pulled off the spec sheet are nomi-

nal, approximate maximum sync frequencies.  The sync pulse in the signal from

the adapter card tells the monitor how fast to actually run.

Recall the two pictures above?  Only part of the time required for raster-

scanning a frame is used for displaying viewable image (ie. your resolution).

9.1  Horizontal Sync:

By previous definition, it takes HFL ticks to trace the a horizontal scan

line.  Let's call the visible tick count (your horizontal screen resolution)

HR.  Then Obviously, HR < HFL by definition.  For concreteness, let's assume

both start at the same instant as shown below:

|___ __ __ __ __ __ __ __ __ __ __ __ __

|_ _ _ _ _ _ _ _ _ _ _ _                |

|_______________________|_______________|_____

0                       ^               ^     unit: ticks

|   ^       ^   |

HR  |       |  HFL

|   |<----->|   |

|<->|  HSP  |<->|

HGT1         HGT2

Now, we would like to place a sync pulse of length HSP as shown above, ie,

between the end of clock ticks for display data and the end of clock ticks

for the entire frame.  Why so?  because if we can achieve this, then your

screen image won't shift to the right or to the left.  It will be where it

supposed to be on the screen, covering squarely the monitor's viewable area.

Furthermore, we want about 30 ticks of "guard time" on either side of the

sync pulse.  This is represented by HGT1 and HGT2.  In a typical configura-

tion HGT1 != HGT2, but if you're building a configuration from scratch, you

want to start your experimentation with them equal (that is, with the sync

pulse centered).

The symptom of a misplaced sync pulse is that the image is displaced on the

screen, with one border excessively wide and the other side of the image

wrapped around the screen edge, producing a white edge line and a band of

"ghost image" on that side.  A way-out-of-place vertical sync pulse can actu-

ally cause the image to roll like a TV with a mis-adjusted vertical hold (in

fact, it's the same phenomenon at work).

If you're lucky, your monitor's sync pulse widths will be documented on its

specification page.  If not, here's where the real black magic starts...

You'll have to do a little trial and error for this part.  But most of the

time, we can safely assume that a sync pulse is about 3.5 to 4.0 microsecond

in length.

For concreteness again, let's take HSP to be 3.8 microseconds (which btw, is

not a bad value to start with when experimenting).

Now, using the 65Mhz clock timing above, we know HSP is equivalent to 247

clock ticks (= 65 * 10**6 * 3.8 * 10^-6) [recall M=10^6, micro=10^-6]

Some makers like to quote their horizontal framing parameters as timings

rather than dot widths.  You may see the following terms:

active time (HAT)

Corresponds to HR, but in milliseconds.  HAT * DCF = HR.

blanking time (HBT)

Corresponds to (HFL - HR), but in milliseconds.  HBT * DCF = (HFL

- HR).

front porch (HFP)

This is just HGT1.

sync time

This is just HSP.

back porch (HBP)

This is just HGT2.

9.2  Vertical Sync:

Going back to the picture above, how do we place the 247 clock ticks as shown

in the picture?

Using our example, HR is 944 and HFL is 1176.  The difference between the two

is 1176 - 944=232 < 247!  Obviously we have to do some adjustment here.  What

can we do?

The first thing is to raise 1176 to 1184, and lower 944 to 936.  Now the dif-

ference = 1184-936= 248. Hmm, closer.

Next, instead using 3.8, we use 3.5 for calculating HSP; then, we have

65*3.5=227.  Looks better.  But 248 is not much higher than 227.  It's nor-

mally necessary to have 30 or so clock ticks between HR and the start of SP,

and the same for the end of SP and HFL.  AND they have to be multiple of

eight!  Are we stuck?

No.  Let's do this, 936 % 8 = 0, (936 + 32) % 8 = 0 too.  But 936 + 32 = 968,

968 + 227 = 1195, 1195 + 32 = 1227.  Hmm.. this looks not too bad.  But it's

not a multiple of 8, so let's round it up to 1232.

But now we have potential trouble, the sync pulse is no longer placed right

in the middle between h and H any more.  Happily, using our calculator we

find 1232 - 32 = 1200 is also a multiple of 8 and (1232 - 32) - 968 = 232

corresponding using a sync pulse of 3.57 micro second long, still reasonable.

In addition, 936/1232 ~ 0.76 or 76%, still not far from 80%, so it should be

all right.

Furthermore, using the current horizontal frame length, we basically ask our

monitor to sync at 52.7khz (= 65Mhz/1232) which is within its capability.  No

problems.

Using rules of thumb we mentioned before, 936*75%=702, This is our new verti-

cal resolution.  702 * 1.05 = 737, our new vertical frame length.

Screen refresh rate = 65Mhz/(737*1232)=71.6 Hz.  This is still excellent.

Figuring the vertical sync pulse layout is similar:

|___ __ __ __ __ __ __ __ __ __ __ __ __

|_ _ _ _ _ _ _ _ _ _ _ _                |

|_______________________|_______________|_____

0                      VR              VFL     unit: ticks

^   ^       ^

|   |       |

|<->|<----->|

VGT    VSP

We start the sync pulse just past the end of the vertical display data ticks.

VGT is the vertical guard time required for the sync pulse.  Most monitors

are comfortable with a VGT of 0 (no guard time) and we'll use that in this

example.  A few need two or three ticks of guard time, and it usually doesn't

hurt to add that.

Returning to the example: since by the definition of frame length, a vertical

tick is the time for tracing a complete HORIZONTAL frame, therefore in our

example, it is 1232/65Mhz=18.95us.

Experience shows that a vertical sync pulse should be in the range of 50us

and 300us.  As an example let's use 150us, which translates into 8 vertical

clock ticks (150us/18.95us~8).

Some makers like to quote their vertical framing parameters as timings rather

than dot widths.  You may see the following terms:

active time (VAT)

Corresponds to VR, but in milliseconds.  VAT * VSF = VR.

blanking time (VBT)

Corresponds to (VFL - VR), but in milliseconds.  VBT * VSF = (VFL

- VR).

front porch (VFP)

This is just VGT.

sync time

This is just VSP.

back porch (VBP)

This is like a second guard time after the vertical sync pulse.

It is often zero.

10.  Putting it All Together

The Xconfig file Table of Video Modes contains lines of numbers, with each

line being a complete specification for one mode of X-server operation.  The

fields are grouped into four sections, the name section, the clock frequency

section, the horizontal section, and the vertical section.

The name section contains one field, the name of the video mode specified by

the rest of the line.  This name is referred to on the "Modes" line of the

Graphics Driver Setup section of the Xconfig file.  The name field may be

omitted if the name of a previous line is the same as the current line.

The dot clock section contains only the dot clock (what we've called DCF)

field of the video mode line.  The number in this field specifies what dot

clock was used to generate the numbers in the following sections.

The horizontal section consists of four fields which specify how each hori-

zontal line on the display is to be generated.  The first field of the sec-

tion contains the number of dots per line which will be illuminated to form

the picture (what we've called HR).  The second field of the section indi-

cates at which dot the horizontal sync pulse will begin.  The third field

indicates at which dot the horizontal sync pulse will end.  The fourth field

specifies the total horizontal frame length (HFL).

The vertical section also contains four fields.  The first field contains the

number of visible lines which will appear on the display (VR).  The second

field indicates the line number at which the vertical sync pulse will begin.

The third field specifies the line number at which the vertical sync pulse

will end.  The fourth field contains the total vertical frame length (VFL).

Example:

#Modename    clock  horizontal timing  vertical timing

"752x564"     40    752 784  944 1088  564 567 569 611

44.5  752 792  976 1240  564 567 570 600

(Note: stock X11R5 doesn't support fractional dot clocks.)

For Xconfig, all of the numbers just mentioned - the number of illuminated

dots on the line, the number of dots separating the illuminated dots from the

beginning of the sync pulse, the number of dots representing the duration of

the pulse, and the number of dots after the end of the sync pulse - are added

to produce the number of dots per line.  The number of horizontal dots must

be evenly divisible by eight.

Example horizontal numbers: 800 864 1024 1088

This sample line has the number of illuminated dots (800) followed by the

number of the dot when the sync pulse starts (864), followed by the number of

the dot when the sync pulse ends (1024), followed by the number of the last

dot on the horizontal line (1088).

Note again that all of the horizontal numbers (800, 864, 1024, and 1088) are

divisible by eight!  This is not required of the vertical numbers.

The number of lines from the top of the display to the bottom form the frame.

The basic timing signal for a frame is the line.  A number of lines will con-

tain the picture.  After the last illuminated line has been displayed, a

delay of a number of lines will occur before the vertical sync pulse is gen-

erated.  Then the sync pulse will last for a few lines, and finally the last

lines in the frame, the delay required after the pulse, will be generated.

The numbers that specify this mode of operation are entered in a manner simi-

lar to the following example.

Example vertical numbers: 600 603 609 630

This example indicates that there are 600 visible lines on the display, that

the vertical sync pulse starts with the 603rd line and ends with the 609th,

and that there are 630 total lines being used.

Note that the vertical numbers don't have to be divisible by eight!

Let's return to the example we've been working.  According to the above, all

we need to do from now on is to write our result into Xconfig as follows:

<name>   DCF     HR  SH1 SH2   HFL   VR  SV1 SV2 VFL

where SH1 is the start tick of the horizontal sync pulse and SH2 is its end

tick; similarly, SV1 is the start tick of the vertical sync pulse and SV2 is

its end tick.

#name    clock   horizontal timing   vertical timing    flag

936x702  65      936 968 1200 1232   702 702 710 737

No special flag necessary; this is a non-interlaced mode.  Now we are really

done.

11.  Overdriving Your Monitor

You should absolutely not try exceeding your monitor's scan rates if it's a

fixed-frequency type.  You can smoke your hardware doing this!  There are

potentially subtler problems with overdriving a multisync monitor which you

should be aware of.

Having a pixel clock higher than the monitor's maximum bandwidth is rather

harmless, in contrast.  (Note: the theoretical limit of discernible features

is reached when the pixel clock reaches double the monitor's bandwidth.  This

is a straightforward application of Nyquist's Theorem: consider the pixels as

a spatially distributed series of samples of the drive signals and you'll see

why.)

It's exceeding the rated maximum sync frequencies that's problematic.  Some

modern monitors might have protection circuitry that shuts the monitor down

at dangerous scan rates, but don't rely on it.  In particular there are older

multisync monitors (like the Multisync II) which use just one horizontal

transformer. These monitors will not have much protection against overdriving

them.  While you necessarily have high voltage regulation circuitry (which

can be absent in fixed frequency monitors), it will not necessarily cover

every conceivable frequency range, especially in cheaper models. This not

only implies more wear on the circuitry, it can also cause the screen phos-

phors to age faster, and cause more than the specified radiation (including

X-rays) to be emitted from the monitor.

Another importance of the bandwidth is that the monitor's input impedance is

specified only for that range, and using higher frequencies can cause reflec-

tions probably causing minor screen interferences, and radio disturbance.

However, the basic problematic magnitude in question here is the slew rate

(the steepness of the video signals) of the video output drivers, and that is

usually independent of the actual pixel frequency, but (if your board manu-

facturer cares about such problems) related to the maximum pixel frequency of

the board.

So be careful out there...

12.  Using Interlaced Modes

(This section is largely due to David Kastrup <dak@pool.informatik.rwth-

aachen.de>)

At a fixed dot clock, an interlaced display is going to have considerably

less noticeable flicker than a non-interlaced display, if the vertical cir-

cuitry of your monitor is able to support it stably.  It is because of this

that interlaced modes were invented in the first place.

Interlaced modes got their bad repute because they are inferior to their non-

interlaced companions at the same vertical scan frequency, VSF (which is what

is usually given in advertisements). But they are definitely superior at the

same horizontal scan rate, and that's where the decisive limits of your moni-

tor/graphics card usually lie.

At a fixed refresh rate (or half frame rate, or VSF) the interlaced display

will flicker more: a 90Hz interlaced display will be inferior to a 90Hz non-

interlaced display. It will, however, need only half the video bandwidth and

half the horizontal scan rate. If you compared it to a non-interlaced mode

with the same dot clock and the same scan rates, it would be vastly superior:

45Hz non-interlaced is intolerable. With 90Hz interlaced, I have worked for

years with my Multisync 3D (at 1024x768) and am very satisfied. I'd guess

you'd need at least a 70Hz non-interlaced display for similar comfort.

You have to watch a few points, though: use interlaced modes only at high

resolutions, so that the alternately lighted lines are close together. You

might want to play with sync pulse widths and positions to get the most sta-

ble line positions. If alternating lines are bright and dark, interlace will

jump at you. I have one application that chooses such a dot pattern for a

menu background (XCept, no other application I know does that, fortunately).

I switch to 800x600 for using XCept because it really hurts my eyes other-

wise.

For the same reason, use at least 100dpi fonts, or other fonts where horizon-

tal beams are at least two lines thick (for high resolutions, nothing else

will make sense anyhow).

And of course, never use an interlaced mode when your hardware would support

a non-interlaced one with similar refresh rate.

If, however, you find that for some resolution you are pushing either monitor

or graphics card to their upper limits, and getting unsatisfactory flickery

or washed out (bandwidth exceeded) display, you might want to try tackling

the same resolution using an interlaced mode. Of course this is useless if

the VSF of your monitor is already close to its limits.

Design of interlaced modes is easy: do it like a non-interlaced mode. Just

two more considerations are necessary: you need an odd total number of verti-

cal lines (the last number in your mode line), and when you specify the

"interlace" flag, the actual vertical frame rate for your monitor doubles.

Your monitor needs to support a 90Hz frame rate if the mode you specified

looks like a 45Hz mode apart from the "Interlace" flag.

As an example, here is my modeline for 1024x768 interlaced: my Multisync 3D

will support up to 90Hz vertical and 38kHz horizontal.

ModeLine "1024x768" 45 1024 1048 1208 1248 768 768 776 807 Interlace

Both limits are pretty much exhausted with this mode. Specifying the same

mode, just without the "Interlace" flag, still is almost at the limit of the

monitor's horizontal capacity (and strictly speaking, a bit under the lower

limit of vertical scan rate), but produces an intolerably flickery display.

Basic design rules: if you have designed a mode at less than half of your

monitor's vertical capacity, make the vertical total of lines odd and add the

"Interlace" flag. The display's quality should vastly improve in most cases.

If you have a non-interlaced mode otherwise exhausting your monitor's specs

where the vertical scan rate lies about 30% or more under the maximum of your

monitor, hand-designing an interlaced mode (probably with somewhat higher

resolution) could deliver superior results, but I won't promise it.

13.  Questions and Answers

Q. The example you gave is not a standard screen size, can I use it?

A. Why not?  There is NO reason whatsoever why you have to use 640x480,

800x600, or even 1024x768.  The XFree86 servers let you configure your hard-

ware with a lot of freedom.  It usually takes two to three tries to come up

the right one.  The important thing to shoot for is high refresh rate with

reasonable viewing area. not high resolution at the price of eye-tearing

flicker!

Q. It this the only resolution given the 65Mhz dot clock and 55Khz HSF?

A. Absolutely not!  You are encouraged to follow the general procedure and do

some trial-and-error to come up a setting that's really to your liking.

Experimenting with this can be lots of fun.  Most settings may just give you

nasty video hash, but in practice a modern multi-sync monitor is usually not

damaged easily. Be sure though, that your monitor can support the frame rates

of your mode before using it for longer times.

Beware fixed-frequency monitors!  This kind of hacking around can damage them

rather quickly. Be sure you use valid refresh rates for every experiment on

them.

Q. You just mentioned two standard resolutions. In Xconfig, there are many

standard resolutions available, can you tell me whether there's any point in

tinkering with timings?

A. Absolutely!  Take, for example, the "standard" 640x480 listed in the cur-

rent Xconfig.  It employs 25Mhz driving frequency, frame lengths are 800 and

525 => refresh rate ~ 59.5Hz. Not too bad.  But 28Mhz is a commonly available

driving frequency from many SVGA boards.  If we use it to drive 640x480, fol-

lowing the procedure we discussed above, you would get frame lengths like 812

and 505.  Now the refresh rate is raised to 68Hz, a quite significant

improvement over the standard one.

Q. Can you summarize what we have discussed so far?

A. In a nutshell:

1.  for any fixed driving frequency, raising max resolution incurs the

penalty of lowering refresh rate and thus introducing more flicker.

2.  if high resolution is desirable and your monitor supports it, try to

get a SVGA card that provides a matching dot clock or DCF. The higher,

the better!

14.  Fixing Problems with the Image.

OK, so you've got your X configuration numbers.  You put them in Xconfig with

a test mode label.  You fire up X, hot-key to the new mode, ... and the image

doesn't look right.  What do you do?  Here's a list of common problems and

how to fix them.

(Fixing these minor distortions is where xvidtune(1) really shines.)

You move the image by changing the sync pulse timing.  You scale it by chang-

ing the frame length (you need to move the sync pulse to keep it in the same

relative position, otherwise scaling will move the image as well).  Here are

some more specific recipes:

The horizontal and vertical positions are independent.  That is, moving the

image horizontally doesn't affect placement vertically, or vice-versa.  How-

ever, the same is not quite true of scaling.  While changing the horizontal

size does nothing to the vertical size or vice versa, the total change in

both may be limited.  In particular, if your image is too large in both

dimensions you will probably have to go to a higher dot clock to fix it.

Since this raises the usable resolution, it is seldom a problem!

14.1  The image is displaced to the left or right

To fix this, move the horizontal sync pulse.  That is, increment or decrement

(by a multiple of 8) the middle two numbers of the horizontal timing section

that define the leading and trailing edge of the horizontal sync pulse.

If the image is shifted left (right border too large, you want to move the

image to the right) decrement the numbers.  If the image is shifted right

(left border too large, you want it to move left) increment the sync pulse.

14.2  The image is displaced up or down

To fix this, move the vertical sync pulse.  That is, increment or decrement

the middle two numbers of the vertical timing section that define the leading

and trailing edge of the vertical sync pulse.

If the image is shifted up (lower border too large, you want to move the

image down) decrement the numbers.  If the image is shifted down (top border

too large, you want it to move up) increment the numbers.

14.3  The image is too large both horizontally and vertically

Switch to a higher card clock speed. If you have multiple modes in your clock

file, possibly a lower-speed one is being activated by mistake.

14.4  The image is too wide (too narrow) horizontally

To fix this, increase (decrease) the horizontal frame length.  That is,

change the fourth number in the first timing section.  To avoid moving the

image, also move the sync pulse (second and third numbers) half as far, to

keep it in the same relative position.

14.5  The image is too deep (too shallow) vertically

To fix this, increase (decrease) the vertical frame length.  That is, change

the fourth number in the second timing section.  To avoid moving the image,

also move the sync pulse (second and third numbers) half as far, to keep it

in the same relative position.

Any distortion that can't be handled by combining these techniques is proba-

bly evidence of something more basically wrong, like a calculation mistake or

a faster dot clock than the monitor can handle.

Finally, remember that increasing either frame length will decrease your

refresh rate, and vice-versa.

15.  Plotting Monitor Capabilities

To plot a monitor mode diagram, you'll need the gnuplot package (a freeware

plotting language for UNIX-like operating systems) and the tool modeplot, a

shell/gnuplot script to plot the diagram from your monitor characteristics,

entered as command-line options.

Here is a copy of modeplot:

#!/bin/sh

#

# modeplot -- generate X mode plot of available monitor modes

#

# Do `modeplot -?' to see the control options.

#

# ($Id: VidModes.sgml,v 3.11.2.2 1998/02/20 23:10:30 dawes Exp $)

# Monitor description. Bandwidth in MHz, horizontal frequencies in kHz

# and vertical frequencies in Hz.

TITLE="Viewsonic 21PS"

BANDWIDTH=185

MINHSF=31

MAXHSF=85

MINVSF=50

MAXVSF=160

ASPECT="4/3"

vesa=72.5 # VESA-recommended minimum refresh rate

while [ "$1" != "" ]

do

case $1 in

-t) TITLE="$2"; shift;;

-b) BANDWIDTH="$2"; shift;;

-h) MINHSF="$2" MAXHSF="$3"; shift; shift;;

-v) MINVSF="$2" MAXVSF="$3"; shift; shift;;

-a) ASPECT="$2"; shift;;

-g) GNUOPTS="$2"; shift;;

-?) cat <<EOF

modeplot control switches:

-t "<description>"  name of monitor            defaults to "Viewsonic 21PS"

-b <nn>             bandwidth in MHz           defaults to 185

-h <min> <max>      min & max HSF (kHz)        defaults to 31 85

-v <min> <max>      min & max VSF (Hz)         defaults to 50 160

-a <aspect ratio>   aspect ratio               defaults to 4/3

-g "<options>"      pass options to gnuplot

The -b, -h and -v options are required, -a, -t, -g optional.  You can

use -g to pass a device type to gnuplot so that (for example) modeplot's

output can be redirected to a printer.  See gnuplot(1) for  details.

The modeplot tool was created by Eric S. Raymond <esr@thyrsus.com> based on

analysis and scratch code by Martin Lottermoser <Martin.Lottermoser@mch.sni.de>

This is modeplot $Revision: 3.11.2.2 $

EOF

exit;;

esac

shift

done

gnuplot $GNUOPTS <<EOF

set title "$TITLE Mode Plot"

# Magic numbers.  Unfortunately, the plot is quite sensitive to changes in

# these, and they may fail to represent reality on some monitors.  We need

# to fix values to get even an approximation of the mode diagram.  These come

# from looking at lots of values in the ModeDB database.

F1 = 1.30 # multiplier to convert horizontal resolution to frame width

F2 = 1.05 # multiplier to convert vertical resolution to frame height

# Function definitions (multiplication by 1.0 forces real-number arithmetic)

ac = (1.0*$ASPECT)*F1/F2

refresh(hsync, dcf) = ac * (hsync**2)/(1.0*dcf)

dotclock(hsync, rr) = ac * (hsync**2)/(1.0*rr)

resolution(hv, dcf) = dcf * (10**6)/(hv * F1 * F2)

# Put labels on the axes

set xlabel 'DCF (MHz)'

set ylabel 'RR (Hz)' 6   # Put it right over the Y axis

# Generate diagram

set grid

set label "VB" at $BANDWIDTH+1, ($MAXVSF + $MINVSF) / 2 left

set arrow from $BANDWIDTH, $MINVSF to $BANDWIDTH, $MAXVSF nohead

set label "max VSF" at 1, $MAXVSF-1.5

set arrow from 0, $MAXVSF to $BANDWIDTH, $MAXVSF nohead

set label "min VSF" at 1, $MINVSF-1.5

set arrow from 0, $MINVSF to $BANDWIDTH, $MINVSF nohead

set label "min HSF" at dotclock($MINHSF, $MAXVSF+17), $MAXVSF + 17 right

set label "max HSF" at dotclock($MAXHSF, $MAXVSF+17), $MAXVSF + 17 right

set label "VESA $vesa" at 1, $vesa-1.5

set arrow from 0, $vesa to $BANDWIDTH, $vesa nohead # style -1

plot [dcf=0:1.1*$BANDWIDTH] [$MINVSF-10:$MAXVSF+20] \

refresh($MINHSF, dcf) notitle with lines 1, \

refresh($MAXHSF, dcf) notitle with lines 1, \

resolution(640*480,   dcf) title "640x480  " with points 2, \

resolution(800*600,   dcf) title "800x600  " with points 3, \

resolution(1024*768,  dcf) title "1024x768 " with points 4, \

resolution(1280*1024, dcf) title "1280x1024" with points 5, \

resolution(1600*1280, dcf) title "1600x1200" with points 6

pause 9999

EOF

Once you know you have modeplot and the gnuplot package in place, you'll need

the following monitor characteristics:

o  video bandwidth (VB)

o  range of horizontal sync frequency (HSF)

o  range of vertical sync frequency (VSF)

The plot program needs to make some simplifying assumptions which are not

necessarily correct.  This is the reason why the resulting diagram is only a

rough description. These assumptions are:

1.   All resolutions have a single fixed aspect ratio AR = HR/VR.  Standard

resolutions have AR = 4/3 or AR = 5/4.  The modeplot programs assumes

4/3 by default, but you can override this.

2.   For the modes considered, horizontal and vertical frame lengths are

fixed multiples of horizontal and vertical resolutions, respectively:

HFL = F1 * HR

VFL = F2 * VR

As a rough guide, take F1 = 1.30 and F2 = 1.05 (see frame (section 8., page

1) "Computing Frame Sizes").

Now take a particular sync frequency, HSF.  Given the assumptions just pre-

sented, every value for the clock rate DCF already determines the refresh

rate RR, i.e. for every value of HSF there is a function RR(DCF).  This can

be derived as follows.

The refresh rate is equal to the clock rate divided by the product of the

frame sizes:

RR = DCF / (HFL * VFL)        (*)

On the other hand, the horizontal frame length is equal to the clock rate

divided by the horizontal sync frequency:

HFL = DCF / HSF               (**)

VFL can be reduced to HFL be means of the two assumptions above:

VFL = F2 * VR

= F2 * (HR / AR)

= (F2/F1) * HFL / AR (***)

Inserting (**) and (***) into (*) we obtain:

RR = DCF / ((F2/F1) * HFL**2 / AR)

= (F1/F2) * AR * DCF * (HSF/DCF)**2

= (F1/F2) * AR * HSF**2 / DCF

For fixed HSF, F1, F2 and AR, this is a hyperbola in our diagram.  Drawing

two such curves for minimum and maximum horizontal sync frequencies we have

obtained the two remaining boundaries of the permitted region.

The straight lines crossing the capability region represent particular reso-

lutions. This is based on (*) and the second assumption:

RR = DCF / (HFL * VFL) = DCF / (F1 * HR * F2 * VR)

By drawing such lines for all resolutions one is interested in, one can imme-

diately read off the possible relations between resolution, clock rate and

refresh rate of which the monitor is capable. Note that these lines do not

depend on monitor properties, but they do depend on the second assumption.

The modeplot tool provides you with an easy way to do this.  Do modeplot -?

to see its control options. A typical invocation looks like this:

modeplot -t "Swan SW617" -b 85 -v 50 90 -h 31 58

The -b option specifies video bandwidth; -v and -h set horizontal and verti-

cal sync frequency ranges.

When reading the output of modeplot, always bear in mind that it gives only

an approximate description. For example, it disregards limitations on HFL

resulting from a minimum required sync pulse width, and it can only be accu-

rate as far as the assumptions are.  It is therefore no substitute for a

detailed calculation (involving some black magic) as presented in Putting it

All Together (section 10., page 1). However, it should give you a better

feeling for what is possible and which tradeoffs are involved.

16.  Credits

The original ancestor of this document was by Chin Fang

<fangchin@leland.stanford.edu>.

Eric S. Raymond <esr@snark.thyrsus.com> reworked, reorganized, and massively

rewrote Chin Fang's original in an attempt to understand it.  In the process,

he merged in most of a different how-to by Bob Crosson

<crosson@cam.nist.gov>.

The material on interlaced modes is largely by David Kastrup <dak@pool.infor-

matik.rwth-aachen.de>

Martin Lottermoser <Martin.Lottermoser@mch.sni.de> contributed the idea of

using gnuplot to make mode diagrams and did the mathematical analysis behind

modeplot.  The distributed modeplot was redesigned and generalized by ESR

from Martin's original gnuplot code for one case.

Generated from XFree86: xc/programs/Xserver/hw/xfree86/doc/sgml/VidModes.sgml,v 3.11.2.1 1998/01/25 04:05:19 dawes Exp $

$XConsortium: VidModes.sgml /main/7 1996/02/21 17:46:17 kaleb $

$XFree86: xc/programs/Xserver/hw/xfree86/doc/VideoModes.doc,v 3.14.2.6 1999/06/25 08:56:59 hohndel Exp $